Grand Valley State University - School of Engineering

2026 Senior Project Proposal

Project Title: __________________Water Heater Vision Testing System_______________

Sponsoring Company:  _____________Bradford White Corporation (BWC)_________________

Company Address:  _________________________________________________________

Technical Contact Name: ________Patrick Ward__________________________________

Technical Contact e-mail:  _________pward@bradfordwhite.com

Technical Contact Phone:  ____________(630)-486-8932_____________________________

Contracting Contact Name:  ___________Kevin Crots______________________________

Contracting Contact e-mail:  ___________kcrost@bradfordwhite.com__________________

Contracting Contact Phone:  ___________________________________________________

The Technical Sponsor (must be a degreed engineer responsible for overseeing technical aspects of project) is responsible for:

- Completing paperwork from GVSU Office of Sponsored Projects if/once project is accepted

- Providing project overview & information

- Meeting with student teams weekly

- Reviewing project progress

- Co-coordinating purchases of materials & equipment

- Attending the final presentation at the Engineering Design Conference in August 2026

- Paying for all agreed upon project costs including, but not limited to:  parts and supplies to build project, fabrication services beyond GVSU facilities/student capabilities, shipping final project to sponsor facility, GVSU senior project sponsorship fee (see fee schedule below)

- Arranging shipment of completed project to sponsor facility

Project Purpose:  Describe problem/opportunity with current product or situation.  Pictures are encouraged and in general the more information provided the better.

Every time a new software release is produced for our water heaters, we must run quality assurance tests on full units to ensure the software functions properly. These tests can take hours upon hours of time to finish, making them costly and time consuming. BWC is looking for a way to automate these tests. BWC has already developed and is implementing testing software for various levels of functional testing, but there is nothing that works with a physical unit and that will detect what is displayed on the appliance itself.

The proposed solution to this problem is a testing system that can be installed onto a fully-assembled, production water heater that will run pre-programmed test scenarios utilizing our current test framework. The test device will operate on the G2 Aerotherm series Bradford White water heaters (pictured below) initially, but should be expandable to future projects that leverage the same Display and Button interface. The device will have to be able to observe and navigate the various menus on the user interface without outside intervention and recognize what is being shown on the screen.

The test device will be used in conjunction with other testing software, so the responsibility of this device will be to navigate the user interface according to a command from an exterior connection interface and report the screen contents back out to that interface.

# Functional Requirements:  What must the product or research achieve?  What are the project objectives and specifications for the final product/process?

- The test system must be capable of being mounted to the front of a with a fully assembled BWC Aerotherm G2 water heater functional display.

- The test device shall be designed in such a way that it can be adjusted with updated designs for new water heater systems.

- The test system must be portable so that it can be removed from a water heater and installed on another unit with minimal alterations to the heater or test device

- The test system must be able to interact with the physical dome push-buttons on the water heater.  The unit shall be able to push the buttons while reading the screen at the same time.

- The test system must be able to navigate the various software menus without user intervention and via commands on its interface

- The test system must recognize what is being shown on the screen and report that back out to the connection interface.

- The test system may have it owns dedicated hardware board that reads the video images and processes them or may use the National Instruments modules depending on the team makeup.  If NI is used, additional scope would be provided to ensure a robust project.

- The test system shall interface with a National Instruments control system for capturing data, analyzing video, and providing feedback.

- The test system shall be a National instruments control system built upon the infrastructure used today for navigating menus.

- The test system shall interface with the our dedicated testing lab NI data collection resources as well as the NI functional test control system.  The exact implementation details can be determined based on the teams capabilities.

# Deliverables:  What is expected by the company at project completion?

- A mechanical design for interfacing with the appliance, starting with the Aerotherm G2 for button activation and holding a vision device that can monitor the screen.

- A working prototype of the device that includes all mechanical parts to hold onto the appliance, interface with the appliance, and all wiring to interface with the NI control and data acquisition systems.

- All embedded and National Instruments source code developed or updated for control.

- All technical documentation needed to recreate the device.

- An instruction manual for using the device.

Special Resources:  Are there special resources needed? For instance:  facilities, material, software, expertise, etc.?

- A existing G2 Aerotherm controller and appliance will be required and will be provided by BWC.

- An I/O interface will be provided for interface with the control for controlling outputs as well as reading a camera.

- Assembly facilities to implement the designed solution mechanically will be needed.  Additive or traditional manufactured is acceptable.  This should be an easily repeatable design for manufacturing.  If needed as a backup, the BWC model shop could be a resource with discussions.

- Skill sets required:

  - Mechanical Design and Integration – Interface design, button interface engagement, repeatable performance, stabilization for the camera.

  - Software – Embedded software and/or national instruments control of the buttons, video analysis, image processing, data analysis, and software design.

Design Challenges:  Describe the design challenges of the proposed project. Will there be any need for novel approaches to solving the problem?

The proposed device is a great example of an embedded product – a combination of physical, electronic, and software needs that must work in conjunction to meet the design criteria. The physical components are constrained to fit an existing design, which provides some unique challenges. The software must be capable of image recognition and report that to an exterior interface. The control and interface and handling may be handled by an embedded microprocessor system or by utilizing the NI frameworks which include video processing.

Proprietary Information and Confidentiality Requirements: What aspects of the project cannot be presented to the public.

The project will contain proprietary software code and testing code.  All software that is custom is the property of Bradford White.  All off the shelf NI software or embedded SDK is the property of the respective entities.

Areas of expertise the student team will likely need and Estimated Team Composition:  typically 3 – 6 persons per team, comprising students in the Computer, Electrical, Mechanical, and Product Design & Manufacturing majors.

The team will need electronics, software, and mechanical expertise.

1  Mechanical Engineers, 1 Electrical Engineer, 2 Computer (Software) Engineers (Electrical engineers would be applicable if they are software focused)

Recommended Team Members: Indicate the names of students (co-op or employee) whom you prefer to have in the project team. The faculty will try to accommodate this recommendation but cannot guarantee student placements on teams.

### Estimated Materials Cost of the Project:  ____$5000______________________________

### All materials to be paid for or provided by the sponsor.  This amount must be approved in writing by the sponsoring company prior to submission of its project proposal and in consultation with the Director of the School of Engineering.  In addition, a sponsorship fee to the Grand Valley State University School of Engineering is required after the students’ design proposal is accepted by the sponsor and the implementation phase of the project is initiated.  This sponsorship fee helps the school to defray the costs associated with the operation of the senior engineering capstone program and execution of sponsored projects.  The 2026 sponsorship fee amounts are shown below:

### 2026 Senior Project Sponsorship Fee Schedule

| Estimated Project Materials Cost | Sponsorship Fee Amount |
| --- | --- |
| $0 – $5,000 | $3200 |
| $5,000 – $10,000 | $4200 |
| $10,000 – $25,000 | $5700 |
| $25,000 – $40,000 | $7200 |
| Greater than $40,000 | Negotiable ~ usually $8200. |

### Please return this form to:

Christopher P. Pung and/or 	Philip Hittepole

Office: 	(616) 331-6262        	(616) 331-6045

Fax: 	(616) 331-7215

email: 	pungc@gvsu.edu  	hittepop@gvsu.edu

Proposed Project Title:  Appliance System Integration Test System

Company Sponsor: Kevin Crots

Technical Sponsor Contact: Patrick Ward

Signature:									Date :