System Level Test Procedure

Residential Heat Pump Water Heaters

V1.5

9/17/2025

Table of Contents

Document History	5

Scope	6

Terms	6

Necessary Equipment	7

Setup Instructions	7

Firmware Version Record	8

Test Procedures	9

User Interface	9

Screen Brightness	9

Screen Timeout	10

Setpoint Lockout	11

Heating	12

Normal Operation – Hybrid Mode	12

Normal Operation – Hybrid Plus Mode	15

Normal Operation – Electric Mode	18

Normal Operation – Heat Pump Only Mode	19

Normal Operation – Vacation Mode	22

CTA Operation – Shed Mode	25

CTA Operation – Loadup Mode	27

CTA Operation – Critical Peak Mode	29

CTA Operation – Grid Emergency Mode	32

TOU Operation - Shed	33

TOU Operation - Loadup	36

CTA Operation - Shed honors minimum compressor on time	40

CTA Operation - Shed honors minimum compressor on time	41

Limp Operation – Hybrid Lower	42

Limp Operation – Heat Pump No electric	44

Limp Operation – Lower Vacation	45

Limp Operation – Upper Only	47

Limp Operation – Lower Only	48

Limp Operation – Hybrid Mode with bad Lower Element	49

Limp Operation – Hybrid Mode with bad Compressor	50

Limp Operation – Electric Mode with bad Upper Element	51

Limp Operation – Electric Mode with bad Lower Element	52

Limp Operation – Electric Mode with bad Lower Element and bad Upper Element	53

Limp Operation – Heat Pump Only with bad Upper Element	54

Limp Operation – Heat Pump Only with bad Lower Element	55

Limp Operation – Heat Pump Only with bad Upper Element and bad Lower Element	56

Limp Operation – Vacation Mode with bad Lower Element	57

Limp Operation – Vacation Mode with bad Compressor	58

Limp Operation – Vacation Mode with bad Compressor and bad Upper Element	59

Limp Operation – Vacation Mode with bad Compressor and bad Lower Element	60

Limp Operation – Vacation Mode with bad Lower Element and bad Upper Element	61

Smart Start	62

Commissioning	64

CTA	64

Communication Timeout	65

Lost/Poor Connection	66

Opt-In Status Change and Temporary Override	67

Mode Priority – CTA, TOU, User Heat Mode	70

Connectivity	72

Bluetooth Connectivity	72

WiFi Provisioning	73

Mobile App Connection	74

Setting Schedules – User Schedules	77

Setting Schedules – TOU Schedules	77

Over the Air Updates	78

OTA Passes – Normal Mode Not Heating	78

OTA Passes – Normal Mode Heating	81

OTA Passes – CTA Mode	84

OTA Passes – TOU Mode	85

OTA Passes –  Firmware Under Test Can Be Applied via OTA	85

OTA Passes – Error Present	87

OTA Fails – Incorrect CEL Download CRC	88

OTA Fails – Incorrect STM Download CRC	91

OTA Fails – Incorrect CEL Image CRC	94

OTA Fails – Incorrect STM Image CRC	97

Final Assembly Testing (FAT) Mode	100

User Settings (CEL) Testing	101

Constants Table	101

Hybrid	101

Hybrid Plus	102

Heat Pump Only	102

Heat Pump No Electric	103

Electric	103

Vacation	103

Hybrid Lower	104

TODO	104

# Document History

| Revision Number | Author Name | Date | Change Summary |
| --- | --- | --- | --- |
| 1.0 | Patrick Ward Joe Girgis | 7/29/2025 | Initial Release |
| 1.1 | Patrick Ward Max Fenwick | 9/17/2025 | Add tests for verifying Canadian default set temp |
| 1.2 | Jacob Kaminski | 10/13/2025 | Add Micro to Micro communication test procedures. |
| 1.3 | Jacob Kaminski | 10/22/2025 | Added fault notification test procedures. |
| 1.4 | Jacob Kaminski | 01/28/2026 | Added Alert notification test procedures. |
| 1.5 | Joseph Girgis | 01/29/2026 | Changed TOU Shed to verify head mode returns to Hybrid instead of user selected heat mode. |

# Scope

This document will describe a set of procedures to verify the functionality of the Aerotherm G2 Heat Pump Water Heater.

# Terms

| Term/Acronym | Meaning |
| --- | --- |
| FAT | Final Assembly Testing |
| DUT | Device Under Test |
| EEV | Electric Expansion Valve |
| EMV | Electronic Mixing Valve |
| STM | The main control microcontroller in charge of relay control and current sensing |
| CEL | The user interface microcontroller in charge of graphical elements, user input, and connectivity |
| BW | Bradford White |
| Test Tool | The Two Hearted Test Tool for viewing Modbus information on the DUT |
| GUI | Graphical User Interface that is connected to the Two Hearted Control |
| T1 | Lower tank temperature |
| T2 | Upper tank temperature |
| CPP | Critical Peak Price |
| HMI | Human Machine Interace |

# Necessary Equipment

- A PC with the following software installed

  - The Two Hearted Test Tool

  - The EPRI CTA Sim Tool

- A RS-485 USB cable that connects to the three pin, J7 connector of the control board

- A RS-485 USB cable that connects to the five pin, J6 connector of the control board

- A mobile phone with the Bradford White Wave App installed

# Setup Instructions

The following steps are necessary to set up the test environment for viewing the necessary information to complete the test procedure. This will ensure that the EPRI Sim tool and the Test Tool can be used throughout the procedure without additional setup steps.

- Ensure both the EPRI tool and Two Hearted Test Tool are installed

- Plug the two RS-485 connectors into J7 and J6 of the Two Hearted Control board and then plug the USB end into the PC used for testing

- Open the EPRI and Two Hearted Test Tool

- In EPRI, select the COM port for the 5 pin RS-485 cable that is plugged into J6 of the control board. CTA only functions on the 5 pin port, so this must be the COM port used in the EPRI tool.

- Select a bit rate of 19200 Bps in EPRI. Then select connect

- EPRI will automatically begin a startup sequence between the tool and the control board (acting as the SGD for CTA). Ensure that EPRI receives messages in response from the control board. There should be acknowledgement messages received by the tool. If timeouts are occurring for the message requests sent from the tool, there is a connection problem or the port selected is incorrect.

- Open the Two Hearted Test Tool

- On the top bar, select the Connection dropdown menu

- Select Connection Setup

- Select the COM port for the 3 pin RS-485 connector.

- Select a Baud rate for Modbus. Most baud rates are acceptable, but 115200 is the most commonly selected rate.

- Click apply

- On the top bar, select the Logging… button

- Select a logging interval and destination folder to save the CSV file to

- Enter a useful file name for the relevant test to be performed

- Select the Enabled checkbox to turn logging on

- Press Ok

  - Logging will begin once the connection button is clicked and stop when the application disconnects from the DUT

- To begin logging, Select the Connection dropdown and press the Connect button

- Ensure that the Test Tool is actively updating data from the control board

# Firmware Version Record

Record the firmware version for the STM and CEL in the table below using the following procedure:

- Power on the device and wait for the loading screen to complete

- Press the Menu Button

- Select the settings option in the menu

- Press the Select Button

- Scroll down using the Down Arrow Button until the “About This Device” option is selected

- Press the Select Button

| Device | FW Version | Commit Hash |
| --- | --- | --- |
| CEL |  |  |
| STM |  |  |

Ensure that the firmware version on the device, as recorded above, matches the firmware version being tested.

# Test Procedures

## User Interface

The following tests will verify that the user interface functions as expected. This include basic interaction functions such as pressing the buttons on the front face, navigating menus, and changing configurable settings.

### Screen Brightness

This test will verify that the screen brightness can be changed

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Press any button on the display to wake it up if screen is black |  |
|  | Press Menu and navigate to Settings -> Display - Brightness |  |
|  | Change the display brightness to 100 using the up/down arrows, then press SELECT. This will be the baseline brightness |  |
|  | Change the display brightness to 80 using the down arrow, then press SELECT |  |
|  | Verify that the screen is dimmer than it was at 100. |  |
|  | Change the display brightness to 40 using the down arrow, then press SELECT |  |
|  | Verify that the screen is dimmer than it was at 80 |  |
|  | Change the display brightness to the lowest amount, then press SELECT |  |
|  | Verify that the screen is dim but not black |  |
|  | Power cycle the unit |  |
|  | Verify that after the unit powers up the screen is still at its lowest brightness setting |  |
|  | Brightness settings can be updated without saving. This time, change the brightness to 100 without pressing select. |  |
|  | Verify that the brightness is at its maximum brightness. This should be true even without saving as the brightness is being previewed. |  |
|  | Press the “BACK” button to navigate out of the brightness control menu |  |
|  | Verify that the brightness dims back to the lowest setting |  |
|  | Navigate back to the brightness control menu and re-set the brightness control to 100 |  |

### Screen Timeout

This test will verify that the screen automatically turns off after

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Press the elect button on the display, then navigate back to the Dashboard Home screen |  |
|  | Navigate to the Display Timeout setting screen by pressing MENU -> Settings -> Display -> Display Timeout |  |
|  | Change the display timeout to 5 minutes. |  |
|  | Wait 5 minutes without interacting with the DUT. Monitor the display during this time. The screen should dim for the last 30 seconds before it goes completely black. |  |
|  | Verify that the screen dims after 4.5 minutes of inactivity |  |
|  | Verify that the screen turns off after 5 minutes of inactivity. |  |
|  | Press any button |  |
|  | Verify that the UI wakes up and the screen is no longer black with a button press |  |
|  | Navigate back to the Display Timeout settings screen. |  |
|  | Change the display timeout to 0 (no timeout) |  |
|  | Wait 5 minutes without interacting with the DUT. Monitor the display during this time. |  |
|  | Verify that after 5 minutes the display did not dim or go black. |  |

### Setpoint Lockout

This test will verify that there is a user lockout for changing the setpoint.

Estimated Time to Complete: 5 minutes

Preconditions:

- There are no active schedules or active DRM events

- The setpoint is 120° F

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Use the arrows on the DUT’s display to decrease the set temperature by 1 degree. Press the select button to accept the new temperature. Repeat this process 4 times for a total of 5 temperature changes. |  |
|  | Verify that the temperature was allowed to be changed 5 times |  |
|  | Attempt to increase the setpoint |  |
|  | Verify that the display informs the user that there is a Control lockout and that changing the setpoint is unavailable. |  |
|  | Decrease the setpoint |  |
|  | Verify that the display allows the user to decrease the setpoint. |  |
|  | Attempt to increase the setpoint |  |
|  | Verify that the display informs the user that there is a Control lockout and that changing the setpoint is unavailable. |  |
|  | Start a time for 5 minutes and wait for the timer to elapse |  |
|  | Attempt to increase the setpoint |  |
|  | Verify that the setpoint lockout is removed after 5 minutes and that the user can increase the setpoint. |  |

### Canadian Location Setting Persists

Estimated Time to Complete: 1 minute

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Press Menu and navigate to SETTINGS->DATE-TIME-LOCATION |  |
|  | Scroll down to and select LOCATION |  |
|  | Press Change and select Canada |  |
|  | Select NEXT |  |
|  | Select NEXT |  |
|  | Select COMPLETE |  |
|  | Press Back 3 times to navigate to the Home Dashboard Screen |  |
|  | Verify that the setpoint temperature is now in Celsius |  |
|  | Verify that the setpoint temperature is 60 C |  |
|  | Power Cycle the Unit and wait for the bootup sequence to finish |  |
|  | Verify that the setpoint temperature is 60 C |  |
|  | Press Menu and navigate to SETTINGS->DATE-TIME-LOCATION |  |
|  |  | Scroll down to and select LOCATION |  |
|  |  | Press Change and select USA |  |
|  |  | Select NEXT |  |
|  |  | Select NEXT |  |
|  |  | Select COMPLETE |  |
|  |  | Press Back 3 times |  |
|  |  | Verify that the setpoint temperature is now in Fahrenheit |  |
|  |  | Verify that the setpoint temperature is 120 F |  |
|  |  | Power Cycle the Unit and wait for the bootup sequence to finish |  |
|  |  | Verify that the setpoint temperature is 120 F |  |

### Canada at Initialization

Estimated Time to Complete: 1 minute

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Perform a full chip erase on the CEL using the Image Tool. |  |
|  | Re-flash the firmware under test. |  |
|  | Power cycle the Controller and wait for the bootup process to complete. |  |
|  | Select Next on the language screen |  |
|  | Select Change on the Location screen and select Canada |  |
|  | Set the appropriate time, date, and timezone settings when prompted. |  |
|  | Select COMPLETE on the Time screen |  |
|  | Verify that the setpoint temperature is now in Celsius |  |
|  | Verify that the setpoint temperature is 60 C (140 F) |  |
|  | Power Cycle the Unit and wait for the bootup process to complete. |  |
|  | Verify that the setpoint temperature is 60 C |  |

### Canadian Location Persists after OTA

Estimated Time to Complete: 15 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Press Menu and navigate to SETTINGS->DATE-TIME-LOCATION |  |
|  | Scroll down to and select LOCATION |  |
|  | Press Change and select Canada |  |
|  | Press select NEXT |  |
|  | Press select NEXT |  |
|  | Select COMPLETE |  |
|  | Press Back 3 times |  |
|  | Verify that the setpoint temperature is now in Celsius |  |
|  | Verify that the setpoint temperature is 60 C |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA Completed |  |
|  | Exit the OTA success screen |  |
|  | Verify that the setpoint temperature is 60 C on the Dashboard Home Screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Press Menu and navigate to SETTINGS->DATE-TIME-LOCATION |  |
|  | Scroll down to and select LOCATION |  |
|  | Verify that the Location listed for the device is Canada |  |
|  | Press back until the settings menu is diaplayed |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL and STM matched the updated versions for the binaries bundled into the OTA binary |  |
|  | In memfault open the test cohort’s page. This can be done by selecting the Cohort under the Release Activations section on the test Release’s page. |  |
|  | Verify that the Version listed for the DUT in memfault is the test bundle version used to OTA the DUT. |  |

## Heating

The following tests will verify that the DUT can heat water in each of the various modes. These tests all require that the tank begins at a setpoint of 120° F and that it is filled with water at the setpoint temperature so that the DUT is not heating.

### Normal Operation – Hybrid Mode

This test will verify the operation of the DUT in hybrid heating mode. It will verify that the DUT will heat water at appropriate times using the appropriate electric loads.

Estimated Time to Complete:

Preconditions:

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- The setpoint is 120° F

| SA | -1 | CSA | 51 | CUA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SB | 46 | CSB | 54 | CUB |  |  |  |
| SC | 5 | CSC | 5 |  |  |  |  |
| SD | 7 | CSD |  |  |  |  |  |
|  |  | CSE | 16 |  |  |  |  |
|  |  | CSF | 300 |  |  |  |  |

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Power on the DUT and allow the start up sequence to complete |  |
|  | Using the display, press the Right arrow key twice to display the Mode Selection Screen |  |
|  | Verify that the display shows the active mode |  |
|  | Verify that there is a Change button |  |
|  | Select the change button |  |
|  | In the change mode screen, select Hybrid mode and press the Select button |  |
|  | Verify that the displayed mode is Hybrid after a few seconds has elapsed |  |
|  | Press the Left Arrow button to navigate the display back to the Dashboard Home Screen |  |
|  | Press the Select Button to display the Status Screen |  |
|  | Draw water from the tank until the Upper Tank temperature displayed in the status page has dropped by 10° F |  |
|  | Verify that the Status screen Shows the Upper Tank Temp decreasing as water is drawn from the tank |  |
|  | Verify that the device status at the top of the screen say “Call for heat” |  |
|  | Press the Back Button |  |
|  | Verify that the Dashboard Home Screen displays the test “Call for Heat” |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start heating State |  |
|  | Verify that the fan relays are on in the right combination |  |
|  | Allow the upper tank temperature to heat up to the set temperature |  |
|  | Wait 5 minutes to let the tank temperature settle |  |
|  | Verify that none of the electric elements turned on during the settling period |  |
|  | Verify that the Compressor Relay turns off in the Test Tool |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State in the Test Tool is Standby |  |
|  | Use the Up Arrow on the display to increase the set temperature to 125° F |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start heating State |  |
|  | Verify that the fan relays are on in the right combination |  |
|  | Allow the upper tank temperature to heat up to the set temperature |  |
|  | Wait 5 minutes to let the tank temperature settle |  |
|  | Verify that none of the electric elements turned on during the settling period |  |
|  | Draw water out of the tank until the upper tank temperature reaches 105° F (20 degrees below set temp) |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start heating State |  |
|  | Wait 5 minutes. If the upper tank temperature would heat to more than 109° F, draw more water so that it remains below that temperature, but at or above 105° F |  |
|  | Verify that after 5 minutes have elapsed the Active State listed in the test tool is the Compressor and Upper state |  |
|  | Verify that the safety relay, the compressor relay, and the upper element relay are all on. |  |
|  | Allow the upper tank temperature to heat up to 122° F |  |
|  | Verify that the Active state transitions to Compressor Top Off state |  |
|  | Verify that the upper element relay turns off |  |
|  | Verify that the compressor relay remains on |  |
|  | Wait for the upper tank temperature to reach 125° F |  |
|  | Verify that all electrical element relays turn off |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State is in the Standby State |  |
|  | Wait 5 minutes for the tank to settle |  |
|  | Verify that the DUT does not begin heating during this settling period |  |

### Normal Operation – Hybrid Plus Mode

This test will verify the state transitions and heating capacity of the Hybrid Plus mode

Estimated Time to Complete:

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- The setpoint is 125° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Power on the DUT and allow the start up sequence to complete |  |
|  | Using the display, press the Right arrow key twice to display the Mode Selection Screen |  |
|  | Verify that the display shows the active mode |  |
|  | Verify that there is a Change button |  |
|  | Select the change button |  |
|  | In the change mode screen, select Hybrid Plus mode and press the Select button |  |
|  | Verify that the displayed mode is Hybrid Plus after a few seconds has elapsed |  |
|  | Press the Left Arrow button to navigate the display back to the Dashboard Home Screen |  |
|  | Press the Select Button to display the Status Screen |  |
|  | Draw water from the tank until T2 displayed in the status page has dropped to at least121° F and T1 has dropped to at least 119° F |  |
|  | Verify the Status screen Shows the Upper Tank Temp decreasing as water is drawn from the tank |  |
|  | Verify that the device status at the top of the screen say “Call for heat” |  |
|  | Press the Back Button |  |
|  | Verify that the Dashboard Home Screen displays the text “Call for Heat” |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Allow the upper tank temperature to heat up to the set temperature |  |
|  | Verify that the Compressor Relay turns off in the Test Tool |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State in the Test Tool is Standby |  |
|  | Wait 5 minutes to let the tank temperature settle |  |
|  | Verify that none of the electric elements turned on during the settling period |  |
|  | Draw water out of the tank until the upper tank temperatureT1 reaches 115° F (20 degrees below set temp) |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Draw water out of the tank until the upper tank temperature reaches 118° F (7° degrees below setpoint) |  |
|  | Verify that after 5 minutes have elapsed the Active State listed in the test tool is the Compressor and Lower state |  |
|  | Verify that the safety relay, the compressor relay, and the lower element relay are all on. |  |
|  | Allow the upper tank temperature to heat up to 122° F |  |
|  | Verify that the Active state transitions to Compressor Top Off state |  |
|  | Verify that the upper element relay turns off |  |
|  | Verify that the compressor relay remains on |  |
|  | Wait for the upper tank temperature to reach 125° F |  |
|  | Verify that all electrical element relays turn off |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State is in the Standby State |  |
|  | Wait 5 minutes for the tank to settle |  |
|  | Change setpoint to 140°F |  |
|  | Draw water until T1 on the status page drops down to 120°F (20 deg below setpoint) |  |
|  | Verify the control is in Compressor&Lower state |  |
|  | Verify that the Safety, Compressor, and Lower Element relay are all on. |  |
|  | Draw water until T2 is at 120° F (20° F below setpoint) |  |
|  | Verify the control transitioned to Compressor&Upper |  |
|  | Verify the safety relay, compressor relay, and upper element relay are all on |  |
|  | Wait for control to reach setpoint |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State is in the Standby State |  |
|  | Wait 5 minutes for the tank to settle |  |
|  | Verify that the DUT does not begin heating during this settling period |  |

### Normal Operation – Electric Mode

This test will verify the state transitions and heating capacity of the Electric Only mode. The DUT will experience three heating cycles – two initiated by drawing water out of the tank until the appropriate tank temperatures are reached and one initiated by raising the setpoint.

Estimated Time to Complete:

| Date | 8/4/2025 |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

Preconditions:

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- The setpoint is 125° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Power on the DUT and allow the start up sequence to complete |  |
| 2. | Using the display, press the Right arrow key once to display the Mode Selection Screen |  |
| 3. | Verify that the display shows the active mode |  |
| 4. | Verify that there is a Change button |  |
| 5. | Select the change button |  |
| 6. | In the change mode screen, select Electric Only mode and press the Select button |  |
| 7. | Verify that the displayed mode is Electric Only after a few seconds has elapsed |  |
| 8. | Press the Left Arrow button to navigate the display back to the Dashboard Home Screen |  |
| 9 | Press the Select Button to display the Status Screen |  |
|  | Using the Test Tool. Navigate to the Mixing Valve page to view T2 compensated and T1 compensated. |  |
| 10. | Draw water until T1 in the TT is 108°F or colder |  |
| 11. | Open the test tool to the test tool to the mixing valve page and verify that the control has transitioned to lower element on. |  |
| 12. | Verify that the safety relay, and lower element relay are on. |  |
| 13. | Draw water until T2 is at least 103 or colder |  |
| 14. | Verify that the control transitioned to Upper child state |  |
| 15. | Verify the safety relay, and upper relay are turned on |  |
| 16. | Let T2 heat up to at least 118° F |  |
| 17. | Verify that the control transitioned to Lower child state |  |
| 18. | Verify the safety relay and, lower relay are on |  |
| 19. | Allow T1 to heat up to at least 115 ° F |  |
| 20. | Verify the control transitioned to the Recovery State |  |
| 21. | Verify that the safety relay is ON, lower relay is ON, and the upper relay is OFF. |  |
| 22. | Wait for T2 to heat up to at least 125.5° F (0.5° F past setpoint) |  |
| 23. | Verify the control has transitioned to Standby state. |  |
| 24. | Verify the safety relay is OFF, upper relay is OFF, and lower relay is OFF. |  |

### Normal Operation – Heat Pump Only Mode

This test will verify that the DUT heats water while in the Heat Pump Only mode. The DUT will experience three heating cycles – two initiated by drawing water out of the tank until the appropriate tank temperatures are reached and one initiated by raising the setpoint.

Estimated Time to Complete:

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

Preconditions:

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- The setpoint is 120° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Power on the DUT and allow the start up sequence to complete |  |
|  | Using the display, press the Right arrow key twice to display the Mode Selection Screen |  |
|  | Verify that the display shows the active mode |  |
|  | Verify that there is a Change button |  |
|  | Select the change button |  |
|  | In the change mode screen, select Heat Pump Only mode and press the Select button |  |
|  | Verify that the displayed mode is Heat Pump Only mode after a few seconds has elapsed |  |
|  | Press the Left Arrow button to navigate the display back to the Dashboard Home Screen |  |
|  | Press the Select Button to display the Status Screen |  |
|  | Draw water from the tank until the lower tank temperature (T1) drops by 27° F |  |
|  | Verify that the Status screen Shows the Upper Tank Temp decreasing as water is drawn from the tank |  |
|  | Verify that the device status at the top of the screen say “Call for heat” |  |
|  | Press the Back Button |  |
|  | Verify that the Dashboard Home Screen displays the text “Call for Heat” |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Allow the upper tank temperature to heat up to the set temperature |  |
|  | Verify that the Compressor Relay turns off in the Test Tool |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State in the Test Tool is Standby |  |
|  | Wait 5 minutes to let the tank temperature settle |  |
|  | Verify that none of the electric elements turned on during the settling period |  |
|  | Use the Up Arrow on the display to increase the set temperature to 121° F |  |
|  | Verify that the Compressor Relay and the Safety Relay turn on |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Allow the upper tank temperature to heat up to the set temperature |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Wait 5 minutes to let the tank temperature settle |  |
|  | Verify that none of the electric elements turned on during the settling period |  |
|  | Draw water out of the tank in a way that makes the upper tank temperature 116° F and the lower tank temperature 114° F. This can be accomplished by drawing the water at a slow rate such that the temperature change is more even between the upper and lower tank. |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Wait for the upper tank temperature to reach the set temperature |  |
|  | Verify that all electrical element relays turn off |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State is in the Standby State |  |
|  | Wait 5 minutes for the tank to settle |  |
|  | Verify that the DUT does not begin heating during this settling period |  |

### Normal Operation – Vacation Mode

This test will verify that the DUT can heat water during Vacation mode. This test will require the drawing water out of the tank until cools to 51° F. Three draw and heat cycles will be performed.

Estimated Time to Complete:

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- The setpoint is 125° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Power on the DUT and allow the start up sequence to complete |  |
|  | Using the display, press the Right arrow key twice to display the Mode Selection Screen |  |
|  | Verify that the display shows the active mode |  |
|  | Verify that there is a Change button |  |
|  | Select the change button |  |
|  | In the change mode screen, select Vacation mode and press the Select button |  |
|  | Verify that the displayed mode is Vacation mode after a few seconds has elapsed |  |
|  | Press the Left Arrow button to navigate the display back to the Dashboard Home Screen |  |
|  | Verify that the set temperature is 54° F |  |
|  | Press the Select Button to display the Status Screen |  |
|  | Draw water from the tank until the upper tank temperature is 51° F |  |
|  | Verify that the Status screen Shows the Upper Tank Temp decreasing as water is drawn from the tank |  |
|  | Verify that the device status at the top of the screen say “Call for heat” |  |
|  | Press the Back Button |  |
|  | Verify that the Dashboard Home Screen displays the text “Call for Heat” |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Allow the upper tank temperature to heat up to the set temperature |  |
|  | Verify that the Compressor Relay turns off in the Test Tool |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State in the Test Tool is Standby |  |
|  | Wait 5 minutes to let the tank temperature settle |  |
|  | Verify that none of the electric elements turned on during the settling period |  |
|  | Draw water from the tank until the upper tank temperature is 51° F |  |
|  | Verify that the Status screen Shows the Upper Tank Temp decreasing as water is drawn from the tank |  |
|  | Verify that the device status at the top of the screen say “Call for heat” |  |
|  | Press the Back Button |  |
|  | Verify that the Dashboard Home Screen displays the text “Call for Heat” |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Allow the upper tank temperature to heat up to the set temperature |  |
|  | Verify that the Compressor Relay turns off in the Test Tool |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State in the Test Tool is Standby |  |
|  | Wait 5 minutes to let the tank temperature settle |  |
|  | Verify that none of the electric elements turned on during the settling period |  |
|  | Draw water from the tank until the upper tank temperature is 51° F |  |
|  | Verify that the Status screen Shows the Upper Tank Temp decreasing as water is drawn from the tank |  |
|  | Verify that the device status at the top of the screen say “Call for heat” |  |
|  | Press the Back Button |  |
|  | Verify that the Dashboard Home Screen displays the text “Call for Heat” |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start Heating State |  |
|  | Allow the upper tank temperature to heat up to the set temperature |  |
|  | Verify that the Compressor Relay turns off in the Test Tool |  |
|  | Verify that the Dashboard Home screen displays “Setpoint Satisfied” |  |
|  | Verify that the Active State in the Test Tool is Standby |  |
|  | Wait 5 minutes to let the tank temperature settle |  |
|  | Verify that none of the electric elements turned on during the settling period |  |

### CTA Operation – Shed Mode

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver |  |
| CEL SW Ver |  |

Preconditions:

- The control is in Electric Only user mode

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 108° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Draw water out of the tank until T2 is below 95° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Hybrid Standard Mode |  |
|  | Verify that the compressor turns on by checking the relays display in the test tool |  |
|  | Verify that after 5 minutes of heating, the upper element turns on |  |
|  | Wait for T2 to rise back to setpoint. |  |
|  | Verify that T2 reached 108° F |  |
|  | Verify that all elements and the compressor turn off. |  |
|  | Wait 5 minutes without interacting with the DUT. |  |
|  | Verify that the DUT did not attempt to heat water during the 5 minute settling time. |  |
|  | Select ‘Run Normal/End Event’ in the EPRI tool. |  |
|  | Verify the user heat mode returns to Electric. |  |
|  | Verify setpoint returns to 115° F |  |
|  | Use the down arrow on the user interface to decrease the user setpoint to 105° F. Press select to accept the new setpoint. |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 105° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Draw water out of the tank until T2 is below 95° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Hybrid Standard Mode |  |
|  | Verify that the compressor turns on by checking the relays display in the test tool |  |
|  | Verify that after 5 minutes of heating, the upper element turns on |  |
|  | Wait for T2 to rise back to setpoint. |  |
|  | Verify that T2 reached 105° F and did not continue heating after reaching the setpoint. |  |
|  | Verify that all elements and the compressor turn off. |  |
|  | Wait 5 minutes without interacting with the DUT. |  |
|  | Verify that the DUT did not attempt to heat water during the 5 minute settling time. |  |
|  | Select ‘Run Normal/End Event’ in the EPRI tool. |  |
|  | Use the up arrow on the user interface to increase the user setpoint to 120° F. Press select to accept the new setpoint. |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, amend the Duration setting to be 10 minutes. Press the ‘Shed Load’ button. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 108° F |  |
|  | Draw water out of the tank until T2 is below 95° F |  |
|  | Verify that the DUT is heating water. |  |
|  | Wait for the 10 minute duration of the shed event to elapse. Make sure that the water temperature during this time does not exceed 105° F |  |
|  | Verify that the DRM icon on the icon bar of the Dashboard Home screen is no longer displayed. |  |
|  | Verify that the system state in the Test Tool returns to Normal Operating state. |  |
|  | Verify that the user heat mode is Electric Mode |  |
|  | Verify that the displayed setpoint on the Dashboard Home screen is 120° F |  |
|  | Verify that when the DRM event ends, the water heater keeps heating. |  |
|  | Wait for the DUT to reach the setpoint of 120° F |  |
|  | Verify that the DUT reaches the setpoint. |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that the DUT does not heat the water heater during the 5 minute settling period. |  |

### CTA Operation – Loadup Mode

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- The control is in Electric Only

- Setpoint is at 115° F

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Load Up button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 115° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Draw water out of the tank until T2 is below 95° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Hybrid Standard Mode |  |
|  | Verify that the compressor turns on by checking the relays display in the test tool |  |
|  | Verify that after 5 minutes of heating, the upper element turns on |  |
|  | Verify that after reaching 99° F, the elements turns off but the compressor remains on |  |
|  | Verify that the heat mode displayed on the Dashboard Home screen is Heat Pump Only Mode |  |
|  | Wait for T2 to rise back to setpoint. |  |
|  | Verify that T2 reached 115° F |  |
|  | Verify that the compressor turns off |  |
|  | Wait 5 minutes without interacting with the DUT. |  |
|  | Verify that the DUT did not attempt to heat water during the 5 minute settling time. |  |
|  | Select ‘Run Normal/End Event’ in the EPRI tool. |  |
|  | Verify the user heat mode returns to Electric. |  |
|  | Verify setpoint is 115° F |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, amend the Duration setting to be 10 minutes. Press the ‘Load Up’ button. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 115° F |  |
|  | Draw water out of the tank until T2 is below 95° F |  |
|  | Verify that the DUT is heating water. |  |
|  | Wait for the 10 minute duration of the load up event to elapse. Make sure that the water temperature during this time does not exceed 100° F |  |
|  | Verify that the DRM icon on the icon bar of the Dashboard Home screen is no longer displayed. |  |
|  | Verify that the system state in the Test Tool returns to Normal Operating state. |  |
|  | Verify that the user heat mode is Electric Mode |  |
|  | Verify that the displayed setpoint on the Dashboard Home screen is 110° F |  |
|  | Verify that when the DRM event ends, the water heater keeps heating. |  |
|  | Wait for the DUT to reach the setpoint of 110° F |  |
|  | Verify that the DUT reaches the setpoint. |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that the DUT does not heat the water heater during the 5 minute settling period. |  |

### CTA Operation – Critical Peak Mode

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

Preconditions:

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- The control is in Electric Only

- Setpoint is at 115° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Initiate CPP Event’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 105° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Draw water out of the tank until T2 is below 100° F |  |
|  | Verify that the compressor turns on by checking the relays display in the test tool |  |
|  | Verify that after 5 minutes of heating, that none of the elements turn on |  |
|  | Wait for T2 to rise back to setpoint. |  |
|  | Verify that T2 reached 105° F |  |
|  | Verify that all elements and the compressor turn off. |  |
|  | Wait 5 minutes without interacting with the DUT. |  |
|  | Verify that the DUT did not attempt to heat water during the 5 minute settling time. |  |
|  | Select ‘Run Normal/End Event’ in the EPRI tool. |  |
|  | Verify the user heat mode returns to Electric Only mode. |  |
|  | Verify setpoint returns to 115° F |  |
|  | Use the down arrow on the user interface to decrease the user setpoint to 100° F. Press select to accept the new setpoint. |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Initiate CPP Event’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 100° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Draw water out of the tank until T2 is below 95° F |  |
|  | Verify that the compressor turns on by checking the relays display in the test tool |  |
|  | Verify that after 5 minutes of heating none of the elements turn on |  |
|  | Wait for T2 to rise back to setpoint. |  |
|  | Verify that T2 reached 100° F and did not continue heating after reaching the setpoint. |  |
|  | Verify that all elements and the compressor turn off. |  |
|  | Wait 5 minutes without interacting with the DUT. |  |
|  | Verify that the DUT did not attempt to heat water during the 5 minute settling time. |  |
|  | Select ‘Run Normal/End Event’ in the EPRI tool. |  |
|  | Use the up arrow on the user interface to increase the user setpoint to 120° F. Press select to accept the new setpoint. |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, amend the Duration setting to be 10 minutes. Press the ‘Initiate CPP Event’ button. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 105° F |  |
|  | Draw water out of the tank until T2 is below 100° F |  |
|  | Verify that the DUT is heating water. |  |
|  | Wait for the 10 minute duration of the critical peak event to elapse. Make sure that the water temperature during this time does not exceed 100° F |  |
|  | Verify that the DRM icon on the icon bar of the Dashboard Home screen is no longer displayed. |  |
|  | Verify that the system state in the Test Tool returns to Normal Operating state. |  |
|  | Verify that the user heat mode is Electric Mode |  |
|  | Verify that the displayed setpoint on the Dashboard Home screen is 120° F |  |
|  | Verify that when the DRM event ends, the water heater keeps heating. |  |
|  | Wait for the DUT to reach the setpoint of 120° F |  |
|  | Verify that the DUT reaches the setpoint. |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that the DUT does not heat the water heater during the 5 minute settling period. |  |

### CTA Operation – Grid Emergency Mode

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Grid Size |  |

Preconditions:

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- The control is in Electric Only

- Setpoint is at 115° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Initiate Grid Emergency’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 54° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Vacation Mode |  |
|  | Draw water out of the tank until T2 is below 90° F |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that the DUT does not begin heating and that none of the elements or the compressor turn on during the 5 minute settling period. |  |
|  | Select ‘Run Normal/End Event’ in the EPRI tool. |  |
|  | Verify the user heat mode returns to Electric Only mode. |  |
|  | Verify setpoint returns to 115° F Note: do not wait for setpoint to be satisfied, as that will impact the ability to complete the next steps (the Upper Temp needs to drop below 90° F within 10 minutes ) |  |
|  | Verify that the DRM symbol on the icon bar of the Dashboard Home screen is no longer displayed |  |
|  | Verify that when the DRM event ends, the water heater begins heating. |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, amend the Duration setting to be 10 minutes. Press the ‘Initiate Grid Emergency’ button. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 54° F |  |
|  | Draw water out of the tank until T2 is below 90° F |  |
|  | Verify that the DUT does not begin heating and that none of the elements or the compressor turn on during the 10 minute DRM event. |  |
|  | Verify that the DRM icon on the icon bar of the Dashboard Home screen is no longer displayed. |  |
|  | Verify that the system state in the Test Tool returns to Normal Operating state. |  |
|  | Verify that the user heat mode is Electric Mode |  |
|  | Verify that the displayed setpoint on the Dashboard Home screen is 115° F |  |
|  | Verify that when the DRM event ends, the water heater begins heating. |  |
|  | Wait for the DUT to reach the setpoint of 115° F |  |
|  | Verify that the DUT reaches the setpoint. |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that the DUT does not heat the water heater during the 5 minute settling period. |  |

### TOU Operation - Shed

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

Preconditions:

- User Heat mode is Electric

- There are no active TOU schedules or DRM events

- The setpoint is set to 115° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\BleTester’. This python script will be used to upload a custom TOU schedule to the controller. |  |
|  | In ‘misc_scripts\SW\BleTester\TestMessages\SystemTestSupport’ open the json file BleCmd_TouShedSchedule.json |  |
|  | Edit the startTime and endTime parameters so that the schedule begins in 5 minutes and lasts for 12 hours total. Save the file. |  |
|  | Open the file ‘misc_scripts\SW\BleTester\main.py’ TODO: Instructions on MAC address setting in the script, forgetting the device in Windows, etc! |  |
|  | Under mainFunc(), make sure the command for SendScheduleSetThenGet is not commented out and that the file path for the Shed Json file is provided as an argument. |  |
|  | From the Dashboard Home screen, navigate one page to the left by selecting the left arrow. |  |
|  | Select the ‘Learn More’ button followed by the ‘Connect’ button. A QR should be displayed |  |
|  | In a command line terminal, run the command: ‘python main.py’ to execute the Ble Script. |  |
|  | On the control board press the “Menu” button |  |
|  | Press the right arrow twice to get to the “Schedules” button. |  |
|  | Select “Manage” |  |
|  | Verify that the ‘Shed TOU Schedule’ is listed as the only TOU schedule on the controller. |  |
|  | Wait for the modified TOU start time to elapse. |  |
|  | Verify that the Dashboard Home screen displays that it is in TOU mode |  |
|  | In the Test Tool, navigate to the CTA / TOU page. |  |
|  | Verify that the system state is TOU mode |  |
|  | Verify that the TOU Manager state is in Shed |  |
|  | Verify that the control is in Heat Pump Only mode |  |
|  | Verify the setpoint is 108° F on the Dashboard Home screen. |  |
|  | Navigate to the Schedules page and verify that the TOU schedule is listed as Active |  |
|  | Start drawing water out of the tank until the lower tank temperature is 81° F. |  |
|  | Verify that the compressor turns on and the unit begins heating |  |
|  | Verify that the Active state listed in the Test Tool is Compressor Start Heating |  |
|  | Allow the DUT to heat such that T2 is above 108° F and T1 is within 5° of T2. |  |
|  | Verify that the compressor turns off. |  |
|  | Verify that the Active state listed in the Test Tool is Standby |  |
|  | Attempt to modify the setpoint temperature on the Dashboard home screen by using the arrow buttons. |  |
|  | Verify that the DUT does not allow the setpoint to be modified during a shed TOU state |  |
|  | Enter the schedule management screen |  |
|  | Press the Manage TOU Schedules button |  |
|  | Next to the current TOU schedule, select the checkmark button so that the TOU schedule becomes disabled. |  |
|  | Navigate to the Dashboard Mode screen |  |
|  | Change the user heat mode to be Hybrid Mode |  |
|  | Verify that the heat mode listed on the Dashboard Home Screen is Electric |  |
|  | Alter the setpoint temperature to be 105° F |  |
|  | Draw water out of the tank until the upper tank temperature, T2, is below 95° F |  |
|  | Navigate to the schedule management screen |  |
|  | Press the Manage TOU Schedules button |  |
|  | Next to the current TOU schedule, select the checkmark button so that the TOU schedule becomes enabled. |  |
|  | Navigate to the Dashboard Home Screen |  |
|  | Verify that the Dashboard Home screen displays that it is in TOU mode |  |
|  | In the Test Tool, navigate to the CTA / TOU page. |  |
|  | Verify that the system state is TOU mode |  |
|  | Verify that the TOU Manager state is in Shed |  |
|  | Verify that the control is in Hybrid Standard Mode |  |
|  | Verify the setpoint is 105° F on the Dashboard Home screen. |  |
|  | Verify that the compressor turns on and the DUT begins heating water |  |
|  | Draw water from the tank until T2 is below 89° F |  |
|  | Keep T2 below 89° F for at least 5 minutes |  |
|  | Verify that the Upper element turns on |  |
|  | Stop drawing water |  |
|  | Allow the DUT to heat water until the upper tank temperature, T2, is greater than 103° F |  |
|  | Verify that the heating mode on the Dashboard Home screen changes to Heat Pump mode |  |
|  | Verify that the upper element is off |  |
|  | Allow the DUT to heat such that T2 is above 105° F and T1 is within 5° of T2. |  |
|  | Verify that the compressor turns off |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that the compressor and both elements remain off during the 5 minute settling period |  |
|  | Verify that the DUT does not heat water above 105° F |  |

### TOU Operation - Loadup

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- User Heat mode is Electric

- There are no active TOU schedules or DRM events

- Setpoint is 115° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\BleTester’. This python script will be used to upload a custom TOU schedule to the controller. |  |
|  | In ‘misc_scripts\SW\BleTester\TestMessages\SystemTestSupport’ open the json file BleCmd_TouLoadUpSchedule.json |  |
|  | Edit the startTime and endTime parameters so that the schedule begins in 5 minutes and lasts for 12 hours total. Save the file. |  |
|  | Open the file ‘misc_scripts\SW\BleTester\main.py’ |  |
|  | Under mainFunc(), make sure the command for SendScheduleSetThenGet is not commented out and that the file path for the Load Up Json file is provided as an argument. TODO: Instructions on MAC address setting in the script, forgetting the device in Windows, etc! |  |
|  | From the Dashboard Home screen, navigate one page to the left by selecting the left arrow. |  |
|  | Select the ‘Learn More’ button followed by the ‘Connect’ button. A QR should be displayed |  |
|  | In a command line terminal, run the command: ‘python main.py’ to execute the Ble Script. |  |
|  | On the control board press the “Menu” button |  |
|  | Press the right arrow twice to get to the “Schedules” button. |  |
|  | Select “Manage” |  |
|  | Verify that the ‘LoadUp TOU Schedule’ is listed as the only TOU schedule on the controller. |  |
|  | Navigate back to the Home screen. |  |
|  | In the Test Tool, verify that the system state is Normal Operation |  |
|  | Wait for the modified TOU start time to elapse. |  |
|  | Verify that the Dashboard Home screen displays that it is in TOU mode |  |
|  | In the Test Tool, navigate to the CTA / TOU page. |  |
|  | Verify that the system state is TOU mode |  |
|  | Verify that the TOU Manager state is in Load Up |  |
|  | Verify that the control is in Hybrid Standard mode |  |
|  | Verify the setpoint is 115° F on the Dashboard Home screen. |  |
|  | Navigate to the Schedules page and verify that the TOU schedule is listed as Active |  |
|  | Draw water out of the tank such that the upper tank temperature decreases by 5° F and the lower tank temperature decreases by 7° F |  |
|  | Verify that the compressor turns on |  |
|  | Verify that the Active state listed in the test tool is Compressor Start Heating state |  |
|  | Draw water until T2 is 99° F or colder. |  |
|  | Wait 5 minutes |  |
|  | Verify the upper element has turned on by checking the Test Tool. |  |
|  | Verify that the Active State listed in the test tool is Compressor and Upper |  |
|  | Allow the DUT to heat such that T2 is above 115° F and T1 is within 5° of T2. |  |
|  | Verify that the elements and the compressor turn off when the DUT reaches the setpoint |  |
|  | Verify that the Active state listed in the test tool is Standby state |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that during the 5 minute settling period the elements and the compressor did not turn on |  |
|  | Navigate to the Schedules page |  |
|  | Verify that the TOU schedule is still active when the DUT is not heating |  |
|  | Draw water out of the tank such that the upper tank temperature decreases by 5° F and the lower tank temperature decreases by 7° F |  |
|  | Verify that the compressor turns on |  |
|  | Verify that the Active state listed in the test tool is Compressor Start Heating state |  |
|  | Verify that the Active state listed in the test tool is Compressor Start Heating state |  |
|  | Draw water until T2 is 99° F or colder. |  |
|  | Wait 5 minutes |  |
|  | Verify the upper element has turned on by checking the Test Tool. |  |
|  | Verify that the Active State listed in the test tool is Compressor and Upper |  |
|  | Wait for the water to heat up to the setpoint. |  |
|  | Allow the DUT to heat such that T2 is above 108° F and T1 is within 5° of T2. |  |
|  | Verify that the Active state listed in the test tool is Standby state |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that during the 5 minute settling period the elements and the compressor did not turn on |  |
|  | Navigate to the Schedules page |  |
|  | Verify that the compressor turns on |  |
|  | Verify that the Active state listed in the test tool is Compressor Start Heating state |  |
|  | Verify that the Active state listed in the test tool is Compressor Start Heating state |  |
|  | Draw water until T2 is 99° F or colder. |  |
|  | Wait 5 minutes |  |
|  | Verify the upper element has turned on by checking the Test Tool. |  |
|  | Verify that the Active State listed in the test tool is Compressor and Upper |  |
|  | Allow the DUT to heat such that T2 is above 108° F and T1 is within 5° of T2. |  |
|  | Verify that the elements and the compressor turn off when the DUT reaches the setpoint |  |
|  | Verify that the Active state listed in the test tool is Standby state |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that during the 5 minute settling period the elements and the compressor did not turn on |  |
|  | Using the arrow buttons on the Dashboard Home screen, change the setpoint temperature to 120° F |  |
|  | Verify that the DUT allows the setpoint to be changed when in a TOU loadup state |  |

### CTA Operation - Shed honors minimum compressor on time

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver |  |
| CEL SW Ver |  |
| Tank Size |  |

Preconditions:

- The control is in Heat Pump Only Mode

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- Setpoint is 120

Notes:

TOU schedules and CTA events that use a ‘shed’ command will lower the setpoint without knowledge of whether or not the compressor has run for the minimum on-time.

This test is very timing specific, so read through the entire procedure before beginning.

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 0. | Have the EPRI tool open. Under UCM Commands, Curtailments. Do not press anything yet, just be ready. |  |
|  | Increase setpoint to 130F |  |
|  | Monitor the Test Tool for the Relay States. Watch for the Safety and Fan relays to come on. As soon as the Compressor relay turns on, proceed to the next step. Verify the Compressor relay turns on. |  |
|  | Press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify the Setpoint Manager is in the “CTA Setpoint” state |  |
|  | Wait up to 5 minutes for the Heat Pump to turn off. Verify that the Heat Pump stayed ON for at least 5 minutes. |  |
|  | Verify the setpoint on the Dashboard is 108F. |  |
|  | Verify that after the compressor turns off |  |
|  | On the EPRI Tool, press the “Run Normal / End Event” button to end the shed event. |  |
|  | Verify that the setpoint on the Dashboard returns to 130F |  |
|  | Verify that the system goes back into Heat Pump mode and begins heating. |  |

### CTA Operation - Shed honors minimum compressor on time

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver |  |
| CEL SW Ver |  |

Preconditions:

- The control is in Heat Pump Only Mode

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

- Setpoint is 120

Notes:

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 0. |  |  |

### Limp Operation – Hybrid Lower

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver |  |
| CEL SW Ver |  |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the upper element relay, and wrap the disconnected end of the wire with electrical tape |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Hybrid Standard. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 17. |  |
| 8. | Verify that the System State has transitioned to Hybrid Lower. |  |
| 9. | Verify the DUT is in Standby state. |  |
| 10. | Verify that the Upper element relay, lower element relay, and compressor relay are off. |  |
| 11. | Draw water until T2 is (110 – SA) and T1 is (110 – SB) Record Target Values: T1: T2: |  |
| 12. | Verify the DUT transitioned to CompressorStartHeating. |  |
| 13. | Verify the compressor relay turned on. |  |
| 14. | Draw water until T2 is (110 – CSA) and T1 is (110 – CSB). Record Target Values: T1: T2: |  |
| 15. | In the test tool navigate to the Mixing Valve Tab and verify the Sealed System State is in EEV_Control. |  |
| 16. | Verify the DUT transitioned to Compressor&Lower child state. |  |
| 17. | Verify the lower element relay, and compressor relay are on. |  |
| 18. | Wait for T1 to reach (110-CLC) Record Target Value: T1: |  |
| 19. | Verify the DUT transitioned to CompressorTopOff. |  |
| 20. | Verify the lower element relay is off, and the compressor relay is on. |  |
| 21. | While compressor is on, wait 10 seconds (CTD). |  |
| 22. | Draw water until T2 is (110 – CTA) and T1 is (110 – CTB) |  |
| 23. | Verify the DUT transitioned back to Compessor&LOwer |  |
| 24. | Verify the lower element relay, and compressor relay are on. |  |
| 25. | Wait for T1 to reach (110-CLC) Record Target Value: T1: |  |
| 26. | Verify the DUT has transitioned to CompressorTopOff. |  |
| 27. | Verify the lower element relay is off, and the compressor relay is on. |  |
| 28. | Wait until T2 heats up to 110, and the difference between T1 and T2 is 5° F (CTC). |  |
| 29. | Verify the DUT has transitioned to Standby state. |  |

### Limp Operation – Heat Pump No electric

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver |  |
| CEL SW Ver |  |
| Tank Size |  |

- Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the upper element relay wire, and the lower element relay wire. |  |
|  | Wrap the disconnected end of the wires with electrical tape |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Hybrid mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 18. |  |
| 8. | Verify that the System State has transitioned to Heat Pump No Electric. |  |
|  | Draw water until T2 is at or below (110 – SC) and T1 is at or below (110 – SD). Record Target Value: T1: T2: |  |
|  | Verify the DUT transitioned to CompressorStartHeating state. |  |
|  | Verify the lower element relay and upper element relay are off, and the compressor relay is on. |  |
|  | Wait for T2 to reach 110° F or warmer. |  |
|  | Verify the DUT has transitioned to Standby state. |  |
|  | Verify the lower element relay, upper element relay, and compressor relay are off. |  |

### Limp Operation – Lower Vacation

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver |  |
| CEL SW Ver |  |
| Tank Size |  |

Preconditions:

- Setpoint is at 54° F

- The temperature of the water is at 54° F

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the upper element relay. |  |
|  | Wrap the disconnected end of the wire with electrical tape. |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Vacation mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 17. |  |
| 8. | Verify that the System State has transitioned to Lower Vacation. |  |
|  | Verify the DUT is in Standby state. |  |
|  | Verify the upper element relay, lower element relay, and compressor relay are off. |  |
|  | Draw water until T2 is at or below (110 – SC) or T1 is at or below (110 – SD). Record Target Value: T1: or T2: |  |
|  | Verify the DUT transitioned to Lower Only |  |
|  | Verify the lower element relay is on and the upper element relay is off and compressor relay is off. |  |
|  | Wait for T2 to reach 110° F or warmer. |  |
|  | Verify the DUT transitioned to Standby. |  |
|  | Verify the Lower element relay, upper element relay and compressor relay are all off. |  |

### Limp Operation – Upper Only

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the lower element relay wire, and compressor relay wire from their relays. |  |
|  | Wrap the disconnected end of the wires with electrical tape. |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Hybrid Standard. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 9. |  |
| 8. | Verify that the System State has transitioned to Upper Only. |  |
|  | Verify the DUT is in Standby state. |  |
|  | Verify the upper element relay, lower element relay, and compressor relay are all off. |  |
|  | Draw water until T2 is (110 – SA) or colder. |  |
|  | Verify the DUT transitions to the Upper child state. |  |
|  | Verify the upper element has turned on. |  |
|  | Wait for T2 to warm up to 110° F or warmer. |  |
|  | Verify the DUT has transitioned to Standby. |  |

### Limp Operation – Lower Only

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the upper element relay wire, and compressor relay wire from their relays. |  |
|  | Wrap the disconnected end of the wires with electrical tape. |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Hybrid Standard. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 10. |  |
| 8. | Verify that the System State has transitioned to Lower Only. |  |
|  | Verify the DUT is in Standby state. |  |
|  | Verify the upper element relay, lower element relay, and compressor relay are all off. |  |
|  | Draw water until T2 is (110 – SA) and T1 is (110 – SB) or colder. Record target values: T1: T2: |  |
|  | Verify the DUT transitions to the Lower Only state. |  |
|  | Verify the lower element has turned on. |  |
|  | Wait for T2 to warm up to 110° F or warmer. |  |
|  | Verify the DUT has transitioned to Standby. |  |

### Limp Operation – Hybrid Mode with bad Lower Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the lower element relay, and wrap the disconnected end of the wire with electrical tape |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Hybrid Standard. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 16. |  |
| 8. | Verify that the System State is still in Hybrid Standard. |  |

### Limp Operation – Hybrid Mode with bad Compressor

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board. |  |
| 2. | Disconnect the compressor element relay wire. |  |
|  | Wrap the disconnected end of the wire with electrical tape. |  |
| 3. | Attach the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Hybrid Standard. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 8. |  |
| 8. | Verify the System State transitions to Electric Only mode. |  |

### Limp Operation – Electric Mode with bad Upper Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

- Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board. |  |
| 2. | Disconnect the upper element relay wire and wrap the disconnected end of the wire with electrical tape. |  |
| 3. | Attach the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Electric Only mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 17. |  |
| 8. | Verify that the System State transitions to Hybrid Lower. |  |

### Limp Operation – Electric Mode with bad Lower Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the lower element relay, and wrap the disconnected end of the wire with electrical tape |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Electric Only mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 16. |  |
| 8. | Verify that the System State is still in Hybrid Standard. |  |

### Limp Operation – Electric Mode with bad Lower Element and bad Upper Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the upper element relay wire, and the lower element relay wire. |  |
|  | Wrap the disconnected end of the wires with electrical tape. |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Electric Only mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 18. |  |
| 8. | Verify that the System State has transitioned to Heat Pump No Electric. |  |

### Limp Operation – Heat Pump Only with bad Upper Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver |  |
| CEL SW Ver |  |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the upper element relay wire. |  |
|  | Wrap the disconnected end of the wire with electrical tape. |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Heat Pump Only mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 17. |  |
| 8. | Verify that the System State stayed in Heat Pump Only mode. |  |

### Limp Operation – Heat Pump Only with bad Lower Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the lower element relay wire. |  |
|  | Wrap the disconnected end of the wire with electrical tape. |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Heat Pump Only mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 17. |  |
| 8. | Verify that the System State stayed in Heat Pump Only mode. |  |

### Limp Operation – Heat Pump Only with bad Upper Element and bad Lower Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver |  |
| CEL SW Ver |  |
| Tank Size |  |

Preconditions:

- Setpoint is at 110° F

- DUT has setpoint satisfied before disconnecting power

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect both the upper element relay wire, and the lower element relay wire. |  |
|  | Wrap the disconnected end of the wires with electrical tape. |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Heat Pump Only mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 17. |  |
| 8. | Verify that the System State stayed in Heat Pump Only mode. |  |

### Limp Operation – Vacation Mode with bad Lower Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 54° F

- The temperature of the water is at 54° F

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the lower element relay, and wrap the disconnected end of the wire with electrical tape |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Vacation mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 16. |  |
| 8. | Verify that the System State has transitioned to Heat Pump No Electric. |  |

### Limp Operation – Vacation Mode with bad Compressor

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 54° F

- The temperature of the water is at 54° F

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the compressor relay, and wrap the disconnected end of the wire with electrical tape |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Vacation mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 17. |  |
| 8. | Verify that the System State has transitioned to Lower Vacation. |  |
|  | Verify the DUT is in Standby state. |  |
|  | Verify the upper element relay, lower element relay, and compressor relay are off. |  |
|  | Draw water until T2 is at or below (110 – SC) or T1 is at or below (110 – SD). Record Target Value: T1: or T2: |  |
|  | Verify the DUT transitioned to Lower Only |  |
|  | Verify the lower element relay is on and the upper element relay is off and compressor relay is off. |  |
|  | Wait for T2 to reach 110° F or greater. |  |
|  | Verify the DUT transitioned to Standby. |  |
|  | Verify the Lower element relay, upper element relay and compressor relay are all off. |  |

### Limp Operation – Vacation Mode with bad Compressor and bad Upper Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 54° F

- The temperature of the water is at 54° F

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the upper element relay wire, and compressor relay wire. |  |
|  | Wrap the disconnected end of the wires with electrical tape |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Vacation mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 10. |  |
| 8. | Verify that the System State has transitioned to Lower Vacation. |  |

### Limp Operation – Vacation Mode with bad Compressor and bad Lower Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 54° F

- The temperature of the water is at 54° F

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the lower element relay wire, and compressor relay wire from their relays. |  |
|  | Wrap the disconnected end of the wires with electrical tape. |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Vacation mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 9. |  |
| 8. | Verify that the System State has transitioned to Upper Only. |  |

### Limp Operation – Vacation Mode with bad Lower Element and bad Upper Element

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| STM SW Ver | 1.17.0.7 |
| CEL SW Ver | 2.0.1.0 |
| Tank Size |  |

Preconditions:

- Setpoint is at 54° F

- The temperature of the water is at 54° F

- DUT is disconnected from power

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
| 1. | Detach the controller housing from the tank to gain access to the control board |  |
| 2. | Disconnect the upper element relay wire, and the lower element relay wire. |  |
|  | Wrap the disconnected end of the wires with electrical tape |  |
| 3. | Attached the controller housing back to the tank. |  |
| 4. | Connect the DUT back to power. |  |
| 5. | Change the user heat mode to Vacation mode. |  |
| 6. | Connect the DUT to the Test Tool via modbus cable. |  |
|  | Wait for DUT to get back to setpoint. |  |
| 7. | In the test tool navigate to the “Mixing Valve” page and verify the Heat Decision Manager has a value of 18. |  |
| 8. | Verify that the System State has transitioned to Heat Pump No Electric. |  |

## Smart Start

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

Preconditions:

- Tank has enough water to cover the lower element, but not covering the upper element

| Step | Action | Result (Pass/Fail) |
| --- | --- | --- |
| 1. | Power on the DUT and connect it to the test tool. |  |
| 2. | Navigate to the “Smart Start and Commissioning” tab and check the “Smart Start Hard Entry Count” checkbox. |  |
| 3. | Press the “Re-enable Smart Start” button. |  |
| 4. | Verify the System State of the DUT is in Smart Start. |  |
| 5. | Verify that Smart Start is in Standby state. |  |
| 6. | Verify the lower element relay, upper element relay, and compressor relay are off. |  |
| 7. | Wait for DUT to transition to Compressor StartHeating state |  |
| 8. | Verify in the Test Tool the compressor relay is turned on |  |
| 9. | Wait for DUT to transition to Upper Element Fault State. |  |
| 10. | Verify the compressor relay is off |  |
| 11. | In the GUI, from Dashboard Home press right arrow until Active Faults has been reached |  |
| 12. | Verify there is an Upper Element Open Fault |  |
| 13. | Wait for DUT to transition to SmartStartComplete State |  |
| 14. | Verify DUT is in SmartStartComplete state |  |
| 15. | Verify lower element relay, upper element relay, and compressor relay are all off. |  |
| 16. | Verify the system state has transitioned to Commissioning. |  |
| 17. | Fill tank with water until the upper element is submerged under 3in of water. |  |
| 18. | Navigate to the “Smart Start and Commissioning” tab and check the “Smart Start Hard Entry Count” checkbox. |  |
| 19. | Press the “Re-enable Smart Start” button. |  |
| 20. | Verify the System State of the DUT is in Smart Start. |  |
| 21. | Verify that Smart Start is in Standby state. |  |
| 22. | Verify the lower element relay, upper element relay, and compressor relay are off. |  |
| 23. | Wait for DUT to transition to Compressor StartHeating state |  |
| 24. | Verify in the Test Tool the compressor relay is turned on |  |
| 25. | Wait for DUT to transition to SmartStartComplete State. |  |
| 26. | Verify DUT is in SmartStartComplete state. |  |
| 27. | Verify lower element relay, upper element relay, and compressor relay are all off. |  |
| 28. | Verify the system state has transitioned to Commissioning. |  |

## Commissioning

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

Preconditions:

- ??

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Power on the DUT |  |
|  | Connect the DUT to the Test Tool via modbus cable |  |
|  | In the test tool navigate |  |

## CTA

### Communication Timeout

This test verifies that after 15 minutes without communication on the CTA interface, an active DRM event will automatically end.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- The control is in Electric Only user mode

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | In the EPRI tool, under UCM Commands, Comm Status, send a comm status of “Found / Good Connection”. Verify that the DUT acknowledges the message using the Serial Data log. |  |
|  | Repeat sending the comm status message every 5 minutes for 30 minutes |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 108° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Stop sending the comm status message |  |
|  | Wait 15 minutes without interacting with the DUT |  |
|  | Verify that the DRM event icon on the Dashboard Home is no longer displayed. |  |
|  | Verify that the System status displayed in the test tool is Normal Operation |  |
|  | Verify that the user heat mode is Electric Mode |  |

### Lost/Poor Connection

This test verifies that after 15 minutes of lost/poor connection (i.e. 15 minutes from the last Good connection status) on the CTA interface, an active DRM event will automatically end.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

Preconditions:

- The control is in Electric Only user mode

- There are no active schedules or active DRM events

- The DUT has reached the setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen | Pass |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) | Pass |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | In the EPRI tool, under UCM Commands, Comm Status, send a comm status of “Found / Good Connection”. Verify that the DUT acknowledges the message using the Serial Data log. | Pass |
|  | Repeat sending the comm status message every 5 minutes for 30 minutes |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen | Pass |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) | Pass |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 108° F | Pass |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Mode | Pass |
|  | Stop sending the Good comm status message. - Note, it is recommended to start a timer from when the last Good Communication message was sent. |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Send a Lost or Poor connection status on the EPRI tool. |  |
|  | Repeat sending the comm status message every 5 minutes for 10 minutes |  |
|  | Verify that the DRM event icon on the Dashboard Home is no longer displayed - This should occur ~15 minutes from the last Good Connection status, and 10 minutes from the first Poor/Lost connection status. | Pass |
|  | Verify that the System status displayed in the test tool is Normal Operation | Pass |
|  | Verify that the user heat mode is Electric Mode | Pass |

### Opt-In Status Change and Temporary Override

Verify that a user changing their Opt-In and Temporary Override status properly interrupts a CTA event.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- The control is in Electric Only user mode

- There are no active schedules or active DRM events

- The setpoint is at 120° F

- The DUT has reached the setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 108° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Draw water out of the tank until the lower tank temp, T1, is below 93° F |  |
|  | Verify that the compressor turns on by checking the relays display in the test tool |  |
|  | Navigate to the settings menu of the DUT by pressing the Menu button and selecting Settings |  |
|  | Enter the “Utility (DRM)” submenu |  |
|  | Press the Select button while the DRM Opted-In item is highlighted to opt-out of DRM events |  |
|  | Verify that the DRM event icon is not longer displayed on the icon bar |  |
|  | Navigate to the Dashboard Home screen |  |
|  | Verify that the setpoint temperature is 120° F |  |
|  | Verify that the user heat mode is Electric |  |
|  | Verify that the Upper element turns on |  |
|  | Allow the water heater to heat water until the upper tank temperature is above 108° F |  |
|  | Verify that the DUT continues heating |  |
|  | Navigate to the settings menu of the DUT by pressing the Menu button and selecting Settings |  |
|  | Enter the “Utility (DRM)” submenu |  |
|  | Press the Select button while the DRM Opted-In item is highlighted to opt-in to DRM events |  |
|  | Navigate to the Dashboard Home Screen |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 108° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Verify that the compressor and all elements turn off |  |
|  | Draw water out of the tank until the lower tank temp, T1, is below 93° F |  |
|  | Verify that the compressor turns on by checking the relays display in the test tool |  |
|  | Navigate to the settings menu of the DUT by pressing the Menu button and selecting Settings |  |
|  | Enter the “Utility (DRM)” submenu |  |
|  | Press the Select button while the Temporary Override item is highlighted |  |
|  | Select a temporary override time of 1 hour |  |
|  | Verify that the DRM event icon is not longer displayed on the icon bar |  |
|  | Navigate to the Dashboard Home screen |  |
|  | Verify that the setpoint temperature is 120° F |  |
|  | Verify that the user heat mode is Electric |  |
|  | Verify that the Upper element turns on |  |
|  | Allow the water heater to heat water until the upper tank temperature is above 108° F |  |
|  | Verify that the DUT continues heating |  |

### Mode Priority – CTA, TOU, User Heat Mode

Verify that a user changing their Opt-In and Temporary Override status properly interrupts a CTA event.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- The control is in Electric Only user mode

- There are no active schedules or active DRM events

- The user setpoint is 115° F

- The DUT has reached the setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\BleTester’. This python script will be used to upload a custom TOU schedule to the controller. |  |
|  | In ‘misc_scripts\SW\BleTester\TestMessages\SystemTestSupport’ open the json file BleCmd_TouLoadUpSchedule.json |  |
|  | Edit the startTime and endTime parameters so that the loadup time begins in 5 minutes and lasts for 15 minutes total. Save the file. |  |
|  | Open the file ‘misc_scripts\SW\BleTester\main.py’ |  |
|  | Under mainFunc(), make sure the command for SendScheduleSetThenGet is not commented out and that the file path for the Load Up Json file is provided as an argument. |  |
|  | From the Dashboard Home screen, navigate one page to the left by selecting the left arrow. |  |
|  | Select the ‘Learn More’ button followed by the ‘Connect’ button. A QR should be displayed |  |
|  | In a command line terminal, run the command: ‘python main.py’ to execute the Ble Script. |  |
|  | On the control board press the “Menu” button |  |
|  | Press the right arrow twice to get to the “Schedules” button. |  |
|  | Select “Manage” |  |
|  | Verify that the ‘Test TOU Schedule’ is listed as the only TOU schedule on the controller. |  |
|  | Verify that the TOU schedule is not labeled as active. |  |
|  | Navigate back to the Home screen. |  |
|  | In the Test Tool, verify that the system state is Normal Operation |  |
|  | Wait for the modified TOU start time to elapse. |  |
|  | Verify that the Dashboard Home screen displays that it is in TOU mode |  |
|  | In the Test Tool, navigate to the CTA / TOU page. |  |
|  | Verify that the system state is TOU mode |  |
|  | Verify that the TOU Manager state is in Load Up |  |
|  | Verify that the control is in Hybrid Standard mode |  |
|  | Verify the setpoint is 115° F on the Dashboard Home screen. |  |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 108° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Heat Pump Only Mode |  |
|  | Draw water from the tank so that the upper tank temp, T2, is 108° F |  |
|  | Verify that none of the elements or the compressor turned on while drawing water. |  |
|  | Wait 5 minutes without interacting with the DUT |  |
|  | Verify that during the 5 minute period, none of the elements or the compressor turned on while drawing water. |  |
|  | Draw water out of the tank until T2 is below 95° F |  |
|  | Verify that the Dashboard Home screen states that the DUT is in Hybrid Standard Mode |  |
|  | Verify that the compressor turns on by checking the relays display in the test tool |  |
|  | Verify that after 5 minutes of heating, the upper element turns on |  |
|  | On the display, navigate to the Dashboard Mode screen |  |
|  | Change the user heat mode to be Vacation mode |  |
|  | Navigate to the Dashboard Home screen |  |
|  | Verify that the user heat mode has changed to Vacation Mode |  |
|  | Verify that the setpoint is 54° F |  |
|  | Verify that the DRM icon on the icon bar is no longer shown. The DRM icon consists of two parallel arrows pointing up and down. |  |
|  | Verify that the system state listed in the test tool is Normal Operation |  |
|  | Verify that the compressor and elements turn off |  |
|  | On the display, navigate to the Dashboard Mode screen |  |
|  | Change the user heat mode to be Electric mode |  |
|  | Navigate to the Dashboard Home screen |  |
|  | Verify that the user heat mode has changed to Electric Mode |  |
|  | Verify that the setpoint is 115° F |  |
|  | Allow the TOU schedule end time to elapse |  |
|  | Verify that the Dashboard Home screen displays that it is in TOU mode |  |
|  | In the Test Tool, navigate to the CTA / TOU page. |  |
|  | Verify that the system state is TOU mode |  |
|  | Verify that the TOU Manager state is in Shed |  |

## Connectivity

### Bluetooth Connectivity

This test verifies the CEL’s ability to connect to the mobile application via Bluetooth.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | From the Home screen, navigate one page to the left. |  |
|  | Verify that the display shows a screen about the BW Wave app |  |
|  | Select ‘Learn More’ and ‘Connect’. |  |
|  | Verify that a QR code to connect to the app with a mobile device is displayed |  |
|  | Switch to the mobile device with the BW Wave app |  |
|  | Select the ‘+’ button in the top right corner. |  |
|  | Select “Choose Appliance” |  |
|  | Select the AeroTherm Series G2 |  |
|  | Select “QR Code is Displayed” |  |
|  | Using the mobile device’s camera, scan the QR code on the Graphical User Interface (GUI) and follow the instructions on Wave app to complete Bluetooth connection. |  |
|  | Verify that the display confirms the mobile app is connected to the DUT |  |

### WiFi Provisioning

This test verifies the CEL’s ability to have Wi-Fi provisioned using the mobile application.

Pre-conditions:

- The control is in Normal Operation Mode.

- Tester has an external device (smart phone) with Bradford White (BW) Wave application and camera.

- The DUT is NOT connected to Wifi

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Verify that the Wifi icon on the Dashboard Home screen’s icon bar has a slash through it indicating it is not connected to Wifi |  |
|  | From the Dashboard Home screen, navigate one page to the left. |  |
|  | Verify that the display shows a screen about the BW Wave app |  |
|  | Select ‘Learn More’ and ‘Connect’. |  |
|  | Verify that a QR code to connect to the app with a mobile device is displayed |  |
|  | In the BW Wave mobile app Select the ‘+’ button in the top right corner. |  |
|  | Select “Choose Appliance” |  |
|  | Select the AeroTherm Series G2 |  |
|  | Select “QR Code is Displayed” |  |
|  | Using the mobile device’s camera, scan the QR code on the GUI and follow the instructions on Wave mobile app to complete Bluetooth connection. |  |
|  | Once connection is established, select “Setup Wi-Fi” |  |
|  | Follow instructions in the mobile app for selecting and entering the password for the selected Wi-Fi network. |  |
|  | Switch back to the control GUI. Navigate to the Wi-Fi screen. |  |
|  | Verify that the control is connected to the control that was selected |  |
|  | Navigate back to the Dashboard Home screen on the DUT’s display |  |
|  | Verify that the Wifi icon on the screen’s icon bar indicates that the DUT is connected to Wifi |  |

### Mobile App Connection

This test verifies that the DUT can communicate date to the mobile application.

Estimated time to completion: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- There are no active schedules or active DRM events

- The DUT is connected to the BW Wave mobile app

- The user has access to the Wave account associated with the DUT

- The setpoint is 120° F

- The user mode is Hybrid Standard mode

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Open the BW Wave mobile application |  |
|  | Open the page in the mobile app for the DUT |  |
|  | Verify that the mode displayed for the DUT in the mobile app is Hybrid Standard |  |
|  | Verify that the mobile app lists the user setpoint is 120° F |  |
|  | In the mobile app, change the setpoint to be 125° F |  |
|  | Verify that the Dashboard Home screen shows a setpoint of 125° F |  |
|  | In the mobile app, change the setpoint to be 130° F |  |
|  | Verify that the Dashboard Home screen shows a setpoint of 130° F |  |
|  | In the mobile app, change the setpoint to be 135° F |  |
|  | Verify that the Dashboard Home screen shows a setpoint of 135° F |  |
|  | In the mobile app, change the setpoint to be 140° F |  |
|  | Verify that the Dashboard Home screen shows a setpoint of 140° F |  |
|  | Use the arrows on the display to change the setpoint of the DUT to be 100° F. Press the Select button to confirm the temperature change. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the setpoint displayed in the mobile app is 100° F |  |
|  | Use the arrows on the display to change the setpoint of the DUT to be 105° F. Press the Select button to confirm the temperature change. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the setpoint displayed in the mobile app is 105° F |  |
|  | Use the arrows on the display to change the setpoint of the DUT to be 110° F. Press the Select button to confirm the temperature change. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the setpoint displayed in the mobile app is 110° F |  |
|  | Use the arrows on the display to change the setpoint of the DUT to be 115° F. Press the Select button to confirm the temperature change. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the setpoint displayed in the mobile app is 115° F |  |
|  | Use the arrows on the display to change the setpoint of the DUT to be 120° F. Press the Select button to confirm the temperature change. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the setpoint displayed in the mobile app is 120° F |  |
|  | Change the user heat mode to Hybrid Plus on the DUT by navigating to the heat mode screen, pressing the change button, and selecting the heat mode. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the user heat mode displayed in the mobile app is Hybrid Plus. |  |
|  | Change the user heat mode to Heat Pump on the DUT by navigating to the heat mode screen, pressing the change button, and selecting the heat mode. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the user heat mode displayed in the mobile app is Heat Pump mode. |  |
|  | Change the user heat mode to Electric on the DUT by navigating to the heat mode screen, pressing the change button, and selecting the heat mode. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the user heat mode displayed in the mobile app is Electric mode. |  |
|  | Change the user heat mode to Vacation on the DUT by navigating to the heat mode screen, pressing the change button, and selecting the heat mode. |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the user heat mode displayed in the mobile app is Vacation mode. |  |
|  | Open the status screen on the DUT by pressing the Select button |  |
|  | Verify that the tank temperature reported by the status screen matches the tank temperature reported by the mobile app. |  |
|  | Draw water from the tank so that the tank temperature drops by 10° F |  |
|  | Reload the DUT’s page on the mobile app |  |
|  | Verify that the tank temperature reported by the status screen matches the tank temperature reported by the mobile app. |  |

### Setting Schedules – User Schedules

This test verifies that the CEL will accept a user type schedules written to the DUT through the mobile application.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Tank Size |  |

Preconditions:

- The DUT is set to Hybrid Standard.

- The DUT has a setpoint of 120 °F and has reached the setpoint.

- All other existing schedules are disabled.

- The BW Wave app is connected to the DUT.

- The DUT has a wifi connection.

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Open the BW Wave App and navigate to the Home screen. |  |
|  | Select “Schedules” and press the “+” in the upper right corner to add a schedule. |  |
|  | Select today as the day the schedule should run. |  |
|  | Set the Start Time to the desired time. Select “Next”. |  |
|  | Set the Mode to Electric and the setpoint to 115. |  |
|  | Set the stop time to 5 minutes after the start time. |  |
|  | Set the ending mode to Hybrid Standard |  |
|  | Name the schedule as Test_1 and save the schedule |  |
|  | Navigate to Schedules in the DUT menu and verify that Test_1 has been added. |  |
|  | Open the BW Wave App and navigate to the home screen. |  |
|  | Select “Schedules” and press the “+” in the upper right corner to add a schedule. |  |
|  | Select today as the day the schedule should run. |  |
|  | Set the Start Time to be the same time as the end of schedule Test_1. Select “Next”. |  |
|  | Set the Mode to Vacation. |  |
|  | Set the stop time to 5 minutes after the start time. |  |
|  | Set the ending mode to Heat Pump with a setpoint of 110° F |  |
|  | Name the schedule as Test_2 and save the schedule |  |
|  | Navigate to Schedules in the DUT menu and verify that Test_2 has been added. |  |

### Setting Schedules – TOU Schedules

This test verifies that the CEL will accept a TOU type schedules written to the DUT through the mobile application.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |

## Over the Air Updates

### OTA Passes – Normal Mode Not Heating

This test will verify that a bundle OTA can be received by the current firmware

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA Completed |  |
|  | Exit the OTA success screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL and STM matched the updated versions for the binaries bundled into the OTA binary |  |
|  | In memfault open the test cohort’s page. This can be done by selecting the Cohort under the Release Activations section on the test Release’s page. |  |
|  | Verify that the Version listed for the DUT in memfault is the test bundle version used to OTA the DUT. |  |

### OTA Passes – Normal Mode Heating

This test will verify that a bundle OTA can be received by the current firmware

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. |  |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Draw water from the tank until the Upper Tank temperature displayed in the status page has dropped by 10° F |  |
|  | Verify that the Dashboard Home Screen displays the test “Call for Heat” |  |
|  | In the Test Tool, open the Mixing Valve tab |  |
|  | Verify that the Compressor Relay and the Safety Relay are on |  |
|  | Verify that the Active State is in the Compressor Start heating State |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes the display informs the user that the Update will install once the tank reaches the setpoint |  |
|  | Wait for the tank to reach the setpoint |  |
|  | Verify that the OTA installation screen, which has a progress bar for installation, does not display while the tank is heating |  |
|  | Verify that once the tank is at setpoint, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA Completed |  |
|  | Exit the OTA success screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL and STM matched the updated versions for the binaries bundled into the OTA binary |  |
|  | In memfault open the test cohort’s page. This can be done by selecting the Cohort under the Release Activations section on the test Release’s page. |  |
|  | Verify that the Version listed for the DUT in memfault is the test bundle version used to OTA the DUT. |  |

### OTA Passes – CTA Mode

This test will verify that a bundle OTA can be received by the current firmware

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The setpoint is 120° F

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Under UCM Commands, Curtailments in the EPRI tool, press the ‘Shed Load’ button. Make sure the duration is set to 0 minutes. |  |
|  | Verify that the DRM icon, consisting of two arrows pointing up and down, is displayed on the icon bar of the Dashboard Home screen |  |
|  | Verify using the test tool that the system state has changed to be the CTA state (state 8) |  |
|  | Verify the Dashboard Home screen of the GUI shows that the setpoint is 108° F |  |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA Completed |  |
|  | Exit the OTA success screen |  |
|  | Verify that the DRM event icon on the icon bar is no longer shown. |  |
|  | Verify that the system state listed in the test tool is Normal Operation |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL and STM matched the updated versions for the binaries bundled into the OTA binary |  |
|  | In memfault open the test cohort’s page. This can be done by selecting the Cohort under the Release Activations section on the test Release’s page. |  |
|  | Verify that the Version listed for the DUT in memfault is the test bundle version used to OTA the DUT. |  |

### OTA Passes – TOU Mode Shed Schedule

This test will verify that a bundle OTA can be received by the current firmware

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\BleTester’. This python script will be used to upload a custom TOU schedule to the controller. |  |
|  | In ‘misc_scripts\SW\BleTester\TestMessages\SystemTestSupport’ open the json file BleCmd_TouShedSchedule.json |  |
|  | Edit the startTime and endTime parameters so that the schedule begins in 5 minutes and lasts for 12 hours total. Save the file. |  |
|  | Open the file ‘misc_scripts\SW\BleTester\main.py’ TODO: Instructions on MAC address setting in the script, forgetting the device in Windows, etc! |  |
|  | Under mainFunc(), make sure the command for SendScheduleSetThenGet is not commented out and that the file path for the Shed Json file is provided as an argument. |  |
|  | From the Dashboard Home screen, navigate one page to the left by selecting the left arrow. |  |
|  | Select the ‘Learn More’ button followed by the ‘Connect’ button. A QR should be displayed |  |
|  | In a command line terminal, run the command: ‘python main.py’ to execute the Ble Script. |  |
|  | On the control board press the “Menu” button |  |
|  | Press the right arrow twice to get to the “Schedules” button. |  |
|  | Select “Manage” |  |
|  | Verify that the ‘Shed TOU Schedule’ is listed as the only TOU schedule on the controller. |  |
|  | Wait for the modified TOU start time to elapse. |  |
|  | Verify that the Schedule icon is displayed on the icon bar |  |
|  | In the Test Tool, navigate to the CTA / TOU page. |  |
|  | Verify that the system state is TOU mode |  |
|  | Verify that the TOU Manager state is in Shed |  |
|  | Verify that the control is in Heat Pump Only mode |  |
|  | Verify the setpoint is 108° F on the Dashboard Home screen. |  |
|  | Navigate to the Schedules page and verify that the TOU schedule is listed as Active |  |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA Completed |  |
|  | Exit the OTA success screen |  |
|  | Verify that the Schedules icon on the icon bar is shown. |  |
|  | Verify that the system state listed in the test tool is in TOU mode |  |
|  | Navigate to the Schedules page and verify that the TOU schedule is listed as Active |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL and STM matched the updated versions for the binaries bundled into the OTA binary |  |
|  | In memfault open the test cohort’s page. This can be done by selecting the Cohort under the Release Activations section on the test Release’s page. |  |
|  | Verify that the Version listed for the DUT in memfault is the test bundle version used to OTA the DUT. |  |

### OTA Passes – TOU Mode Loadup Schedule

This test will verify that a bundle OTA can be received by the current firmware

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

- The tank setpoint is 120° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\BleTester’. This python script will be used to upload a custom TOU schedule to the controller. |  |
|  | In ‘misc_scripts\SW\BleTester\TestMessages\SystemTestSupport’ open the json file BleCmd_TouLoadupSchedule.json |  |
|  | Edit the startTime and endTime parameters so that the schedule begins in 5 minutes and lasts for 12 hours total. Save the file. |  |
|  | Open the file ‘misc_scripts\SW\BleTester\main.py’ TODO: Instructions on MAC address setting in the script, forgetting the device in Windows, etc! |  |
|  | Under mainFunc(), make sure the command for SendScheduleSetThenGet is not commented out and that the file path for the Shed Json file is provided as an argument. |  |
|  | From the Dashboard Home screen, navigate one page to the left by selecting the left arrow. |  |
|  | Select the ‘Learn More’ button followed by the ‘Connect’ button. A QR should be displayed |  |
|  | In a command line terminal, run the command: ‘python main.py’ to execute the Ble Script. |  |
|  | On the control board press the “Menu” button |  |
|  | Press the right arrow twice to get to the “Schedules” button. |  |
|  | Select “Manage” |  |
|  | Verify that the ‘Loadup TOU Schedule’ is listed as the only TOU schedule on the controller. |  |
|  | Wait for the modified TOU start time to elapse. |  |
|  | Verify that the Schedule icon is displayed on the icon bar |  |
|  | In the Test Tool, navigate to the CTA / TOU page. |  |
|  | Verify that the system state is TOU mode |  |
|  | Verify that the TOU Manager state is in Load Up |  |
|  | Verify that the control is in Hybrid Only mode |  |
|  | Navigate to the Schedules page and verify that the TOU schedule is listed as Active |  |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA Completed |  |
|  | Exit the OTA success screen |  |
|  | Verify that the Schedules icon on the icon bar is shown. |  |
|  | Verify that the system state listed in the test tool is in TOU mode |  |
|  | Navigate to the Schedules page and verify that the TOU schedule is listed as Active |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL and STM matched the updated versions for the binaries bundled into the OTA binary |  |
|  | In memfault open the test cohort’s page. This can be done by selecting the Cohort under the Release Activations section on the test Release’s page. |  |
|  | Verify that the Version listed for the DUT in memfault is the test bundle version used to OTA the DUT. |  |

### OTA Passes – Firmware Under Test Can Be Applied via OTA

This test will verify that the firmware under test can be applied as an OTA on previous versions of the firmware.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Previous STM Version |  |
| Previous CEL Version |  |

Preconditions:

- The previous release of FW for the STM and CEL is running on the DUT

  - Mark those FW versions in the test info box above

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Ensure that the FW release under test has been uploaded to MemFault as an OTA release |  |
|  | Add the cohort for the DUT to the release in MemFault |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA Completed |  |
|  | Exit the OTA success screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL and STM matched the updated versions for the binaries bundled into the OTA binary |  |
|  | In memfault open the test cohort’s page. This can be done by selecting the Cohort under the Release Activations section on the test Release’s page. |  |
|  | Verify that the Version listed for the DUT in memfault is the test bundle version used to OTA the DUT. |  |

### OTA Passes – Error Present

This test will verify that a bundle OTA can be received by the current firmware

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

- The tank setpoint is 120° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Detach the controller housing from the tank to gain access to the control board. |  |
|  | Disconnect the compressor element relay wire, the upper element relay wire, and the lower element relay wire. |  |
|  | Wrap the disconnected end of the wire with electrical tape. |  |
|  | Attach the controller housing back to the tank. |  |
|  | Connect the DUT back to power. |  |
|  | Allow the DUT to boot into Dashboard Home screen |  |
|  | Draw water from the tanks until T2 Reaches 90° F |  |
|  | Verify that within 10 minutes an upper element, lower element, and compressor errors are displayed. The errors may be displayed before reaching a tank temperature of 90° F and may not occur at the same time. |  |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. |  |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA Completed |  |
|  | Exit the OTA success screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL and STM matched the updated versions for the binaries bundled into the OTA binary |  |
|  | In memfault open the test cohort’s page. This can be done by selecting the Cohort under the Release Activations section on the test Release’s page. |  |
|  | Verify that the Version listed for the DUT in memfault is the test bundle version used to OTA the DUT. |  |
|  | Power off the DUT |  |
|  | Reattach the compressor, upper element, and lower element connectors to the back of the board. |  |

### OTA Fails – Incorrect CEL Download CRC

This test verifies that the DUT checks the CRC of the download contained in the OTA header for the CEL section of the binary bundle.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Open the CEL binary in a hex editor |  |
|  | Locate the CRC field at the bottom of the binary file. For the CEL it will be located in memory location 2F9FFC. |  |
|  | Edit the CRC field to a new value, which corrupts the CRC. Save the file. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open the CEL binary in a hex editor |  |
|  | Locate the CRC field at the bottom of the CEL binary. For the CEL it will be located in memory location 2FA03C. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Make sure that the MQTT traffic for the DUT is being recorded for review. It will be checked after the OTA finishes. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up. |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA failed |  |
|  | Exit the OTA failure screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the STM matches the updated version for the OTA binary. |  |
|  | Verify that the displayed CEL version is the same version as the release. |  |
|  | In MQTT Explorer, look at the history of the “AeroTherm/[MAC]/System/UpdateStatus” topic. |  |
|  | Verify that the DUT reported a status of 7 during the OTA process, indicating the CEL failed to download. |  |

### OTA Fails – Incorrect STM Download CRC

This test verifies that the DUT checks the CRC of the download contained in the OTA header for the STM section of the binary bundle.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Open the CEL binary in a hex editor |  |
|  | Locate the CRC field at the bottom of the binary file. For the CEL it will be located in memory location 2F9FFC. |  |
|  | Edit the CRC field to a new value, which corrupts the CRC. Save the file. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open the CEL binary in a hex editor |  |
|  | Locate the CRC field at the bottom of the STM binary. For the STM it will be located in memory location 359F50. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Make sure that the MQTT traffic for the DUT is being recorded for review. It will be checked after the OTA finishes. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up. |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA failed |  |
|  | Exit the OTA failure screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the STM matches the updated version for the OTA binary. |  |
|  | Verify that the displayed CEL version is the same version as the release. |  |
|  | In MQTT Explorer, look at the history of the “AeroTherm/[MAC]/System/UpdateStatus” topic. |  |
|  | Verify that the DUT reported a status of 8 during the OTA process, indicating the STM failed to download. |  |

### OTA Fails – Incorrect CEL Image CRC

This test verifies that the DUT checks the CRC of the CEL image contained in the OTA before applying the binary.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Open the CEL binary in a hex editor |  |
|  | Locate the CRC field at the bottom of the binary file. For the CEL it will be located in memory location 2F9FFC. |  |
|  | Edit the CRC field to a new value, which corrupts the CRC. Save the file. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Make sure that the MQTT traffic for the DUT is being recorded for review. It will be checked after the OTA finishes. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up. |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA failed |  |
|  | Exit the OTA failure screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the STM matches the updated version for the OTA binary. |  |
|  | Verify that the displayed CEL version is the same version as the release. |  |
|  | In MQTT Explorer, look at the history of the “AeroTherm/[MAC]/System/UpdateStatus” topic. |  |
|  | Verify that the DUT reported a status of 11 during the OTA process, indicating the CEL failed to be applied. |  |

### OTA Fails – Incorrect STM Image CRC

This test verifies that the DUT checks the CRC of the STM image contained in the OTA before applying the binary.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| New STM Version |  |
| New CEL Version |  |
| Bundle Version |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Locate the signed STM binary and the CEL binary. You will need copies without the OTA headers on them. Copy them into an accessible folder. |  |
|  | Download the misc_scripts folder from DevOps. |  |
|  | In a command line terminal, navigate to ‘misc_scripts\SW\OTA’. |  |
|  | Locate the FW version for both the STM and CEL. |  |
|  | Select a new FW version for the minor field of the STM binary and the dev field of the CEL binary. In order to receive an OTA, the updated FW version must not match the version currently on the DUT. Record the new expected FW version. | New FW Ver: |
|  | In the command line window at the ‘misc_scripts\SW\OTA’ folder, run the VersionEditor.py script using python on both the CEL and STM binaries. The CEL binary will only require the input file and the new dev version selected above as arguments. The STM binary will require the input file location, the new minor FW version, the private signature key, and the public signature key. |  |
|  | The output files will be STM_SIGNED_X_X_X_X.bin and CEL_X_X_X_X.bin where the X’s are the updated FW version. |  |
|  | Open the STM binary in a hex editor |  |
|  | Locate the CRC field at the bottom of the binary file. For the STM it will be located in memory location 5FEFC. |  |
|  | Edit the CRC field to a new value, which corrupts the CRC. Save the file. |  |
|  | Edit the ‘misc_scripts\SW\OTA\config.json’ file to include the new STM and CEL binaries. The bundle version should not include 255 for any digit and should match the new FW number used in the STM and CEL binaries. The STM_application_path and CEL_km4_km0_path should point to the updated STM and CEL binaries respectively. The rest of the fields will not be used, but need to point to valid files. |  |
|  | In the command line, run the GenerateRelease.py script using python. This will generate a binary bundle that can be received by the DUT through an OTA. |  |
|  | Locate the OTA bundle file created using the GenerateRelease script. It will be located in the Release folder under ‘RELEASE_X.X.X.X\OTA_BUNDLE_X.X.X.X’ where X.X.X.X stands for the bundle version provided to the GenerateRelease script in the config.json file. |  |
|  | Open MemFault. |  |
|  | Select the ‘OTA Releases’ tab on the menu bar on the left-hand side of the web page |  |
|  | Select the ‘Create Release’ button. |  |
|  | Enter the bundle Version under Version and add a note to specify that this is a test OTA. |  |
|  | Click the create button. |  |
|  | Open the new release by clicking on the release version number in the OTA Releases page. |  |
|  | Press the Activate button. |  |
|  | Add a cohort that contains only the DUT and any other devices used for OTA testing. |  |
|  | Make sure the type selected is Normal |  |
|  | Press Activate |  |
|  | On the Release page, select the “Add OTA Payload to Release” button. |  |
|  | Select the hardware version from the dropdown. The version selected should be “235-55208-00-0B” for the Two Hearted Control board. |  |
|  | Under upload, press the Select File button and upload the OTA bundle binary created above. |  |
|  | Press Add |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Make sure that the MQTT traffic for the DUT is being recorded for review. It will be checked after the OTA finishes. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | The content of the message should be “m” without quotes. Select the Publish Button. |  |
|  | Monitor the display of the DUT. The screen should transition to an OTA update screen within 30 seconds |  |
|  | Verify that the DUT informs the user that the OTA is downloading |  |
|  | Verify that within 10 minutes, the display shows that the OTA update is installing |  |
|  | Verify that there is a progress bar that increments over time as the OTA progresses. |  |
|  | Wait for the OTA to finish |  |
|  | Verify that shortly after the updating bar is full the display will turn off indicating the controller is resetting. The display may show the Dashboard home screen briefly. |  |
|  | Wait for the controller to boot up. |  |
|  | Verify that once the controller completes booting the DUT displays a screen stating that the OTA failed |  |
|  | Exit the OTA failure screen |  |
|  | Press the menu button and enter the settings menu |  |
|  | Scroll down to select the About Device submenu |  |
|  | Verify that the FW versions displayed for the CEL matches the updated version for the OTA binary. |  |
|  | Verify that the displayed STM version is the same version as the release. |  |
|  | In MQTT Explorer, look at the history of the “AeroTherm/[MAC]/System/UpdateStatus” topic. |  |
|  | Verify that the DUT reported a status of 12 during the OTA process, indicating the STM failed to be applied. |  |

### OTA Rejected – Bad URL

This test verifies that the DUT can handle and reject OTAs from bad URLs.

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |

Preconditions:

- There are no active DRM events or schedules

- The DUT is in Normal Operation

- The tank has reached setpoint

- The tank setpoint is 120° F

| Step | Action | Result (Pass / Fail) |
| --- | --- | --- |
|  | Open a serial terminal window. TeraTerm or ucon is recommended. |  |
|  | Connect an RS-232 serial cable to J17 of the control board, which is the serial port for the CEL. |  |
|  | In the serial terminal window, connect to the comm port for the RS232 cable. Setup the serial settings to use the following parameters: Baud Rate: 115200 Data: 8 bit Parity: none Stop bits: 1 bit |  |
|  | Open MQTT Explorer |  |
|  | Connect to the development broker used by the DUT to connect to the cloud. |  |
|  | Make sure that the MQTT traffic for the DUT is being recorded for review. It will be checked after the OTA finishes. |  |
|  | Use the publish menu on the right-hand side. Enter the following under the Topic: “AeroTherm/[MAC]/System/Update” Where [MAC] is the MAC address of the DUT |  |
|  | Publish the following message to the DUT: { "CEL": { }, "STM": { }, "Upgrade_Bundle_Version": "0.0.0.255", "Upgrade_Bundle_Path": "https://cuscommonstaticdev.z19.web.core.windows.net//TwoHeartedDev/OTA_BUNDLE_0.0.0.255.bin" } |  |
|  | Verify that the DUT writes an Http 404 error to the serial terminal |  |

## Micro to Micro communication

The following test(s) will verify that communication loss between the STM and CEL chips is correctly handled. The tests include basic interaction functions such as pressing the keys on the front face, navigating menus, and changing configurable settings. The tests also include more involved interactions, such as using the ImageTool (CEL) and STM32CubeProgrammer (STM) to flash and erase code. The TwoHearted Test Tool will also be utilized.

### FFT1 - M2M communication loss.

This test will verify that a special diagnostic screen is displayed during FFT1 when the CEL loses communication with the STM. If communication is re-established while on this diagnostic screen, the display will transition back to the correct workflow.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication screen.

- STM programmed with software that enables FFT1 (example: 1.25.1.10).

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the STM32 programmer to your PC and to the correct header on the controller. | -- |
| 2. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 3. | Power on the DUT. | -- |
| 4. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 5. | Verify that the STM is in FFT1 mode via the test tool. | Pass or Fail goes here. |
| 6. | Verify that the DUT’s display is dark. | Pass or Fail goes here. |
| 7. | Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 8. | Verify that after ~12 seconds, the Diagnostic Communication Failure screen is displayed on the DUT. | Pass or Fail goes here. |
| 9. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 10. | Verify that the DUT’s display leaves the Failure screen and goes dark. | Pass or Fail goes here. |

### FFT2 - M2M communication loss.

This test will verify that a special diagnostic screen is displayed during FFT2 when the CEL loses communication with the STM. If communication is re-established while on this diagnostic screen, the display will transition back to the correct workflow.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication screen.

- STM programmed with software that enables FFT2 (example: 1.25.3.10).

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the STM32 programmer to your PC and to the correct header on the controller. | -- |
| 2. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 3. | Power on the DUT. | -- |
| 4. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 5. | Verify that the STM is in FFT2 mode via the test tool. | Pass or Fail goes here. |
| 6. | Verify that the DUT’s display is dark. | Pass or Fail goes here. |
| 7. | Via the Test Tool, step thru FFT2 until Step 5 is reached. | -- |
| 8. | Verify the DUT’s display is no longer dark (it should be red). - This provides visual confirmation that the CEL is in FFT2 mode. | Pass or Fail goes here. |
| 9. | Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 10. | Verify that after ~12 seconds, the Diagnostic Communication Failure screen is displayed on the DUT. | Pass or Fail goes here. |
| 11. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 12. | Verify that the DUT’s display leaves the Failure screen and goes dark. | Pass or Fail goes here. |
| 13. | Via the Test Tool, step thru FFT2 until Step 5 is reached. | -- |
| 14. | Verify the DUT’s display is no longer dark (it should be red). - This provides visual confirmation that the CEL is in FFT2 mode. | Pass or Fail goes here. |

### FAT - M2M communication loss.

This test will verify that a special diagnostic screen is displayed during FAT when the CEL loses communication with the STM. If communication is re-established while on this diagnostic screen, the display will transition back to the correct workflow.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication screen.

- STM programmed with software that enables FAT (example: 1.25.3.10).

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the STM32 programmer to your PC and to the correct header on the controller. | -- |
| 2. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 3. | Power on the DUT. | -- |
| 4. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 5. | Verify that the STM is in FAT mode via the test tool. - If it is not in FAT mode, use the available Test Tool commands to enter FAT, then power cycle the DUT and check for FAT mode again. | Pass or Fail goes here. |
| 6. | Verify that the DUT’s display shows the FAT screen (should have a giant QR code). | Pass or Fail goes here. |
| 7. | Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 8. | Verify that after ~12 seconds, the Diagnostic Communication Failure screen is displayed on the DUT. | Pass or Fail goes here. |
| 9. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 10. | Verify that the DUT’s display leaves the Failure screen and goes back to the FAT screen. | Pass or Fail goes here. |

### Field Replacement - M2M communication loss.

This test will verify that a special diagnostic screen is displayed during FAT (specifically when the CEL has Field Replacement enabled) when the CEL loses communication with the STM. If communication is re-established while on this diagnostic screen, the display will transition back to the correct workflow.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication screen.

- STM programmed with software that enables FAT (example: 1.25.3.10).

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the STM32 programmer to your PC and to the correct header on the controller. | -- |
| 2. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 3. | Power on the DUT. | -- |
| 4. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 5. | Verify that the STM is in FAT mode via the test tool. - If it is not in FAT mode, use the available Test Tool commands to enter FAT, then power cycle the DUT and check for FAT mode again. | Pass or Fail goes here. |
| 6. | Verify that the DUT’s display shows the FAT screen (should have a giant QR code). | Pass or Fail goes here. |
| 7. | Press and hold the BACK and MENU keys on the DUT for at least 10 seconds or until the Field Replacement box turns green on the display. | -- |
| 8, | Power Cycle the DUT. | -- |
| 9. | Verify that the DUT’s display shows the Field Replacement workflow (the first screen should be for Tank Size selection). | Pass or Fail goes here. |
| 10. | Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 11. | Verify that after ~12 seconds, the Diagnostic Communication Failure screen is displayed on the DUT. | Pass or Fail goes here. |
| 12. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 13. | Verify that the DUT’s display leaves the Failure screen and goes back to the Field Replacement workflow. | Pass or Fail goes here. |

### Initial Setup and SmartStart - M2M communication loss

This test will verify that a special diagnostic screen is displayed during initial setup (specifically when the Water Heater is installed and powered up for the first time) when the CEL loses communication with the STM. If communication is re-established while on this diagnostic screen, the display will transition back to the correct workflow.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication screen, and the initial startup flag has been cleared (this can be done by erasing and flashing the CEL with the ImageTool Programming CEL with TTL 232 ).

- STM programmed with software that enables SmartStart (example: 1.25.3.10).

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the STM32 programmer to your PC and to the correct header on the controller. | -- |
| 2. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 3. | Power on the DUT. | -- |
| 4. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 5. | Verify that the STM is in SmartStart via the test tool. - If it is not in SmartStart, use the available Test Tool commands to enter SmartStart, then power cycle the DUT and check for SmartStart again. | Pass or Fail goes here. |
| 6. | Verify that the DUT’s display shows the Initial Setup workflow (the first screen is Language selection). | Pass or Fail goes here. |
| 7. | Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 8. | Verify that after ~12 seconds, the Diagnostic Communication Failure screen is displayed on the DUT. | Pass or Fail goes here. |
| 9. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 10. | Verify that the DUT’s display leaves the Failure screen and goes back to the initial setup workflow. | Pass or Fail goes here. |
| 11. | Complete the setup workflow on the DUT, but do not bypass the SmartStart screen. | -- |
| 12. | Verify the DUT’s display shows the SmartStart screen. | Pass or Fail goes here. |
| 13. | Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 14. | Verify that after ~12 seconds, the Diagnostic Communication Failure screen is displayed on the DUT. | Pass or Fail goes here. |
| 15. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 16 | Verify that the DUT’s display leaves the Failure screen and goes back to the SmartStart screen. | Pass or Fail goes here. |

### Normal Operation (power up) - M2M communication loss.

This test will verify that a special diagnostic screen is displayed when the CEL loses communication with the STM at any point during the startup sequence. If communication is re-established while on this diagnostic screen, the display will transition back to the correct workflow.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication screen.

- STM programmed with software that is in Normal Operation.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the STM32 programmer to your PC and to the correct header on the controller. | -- |
| 2. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 3. | Power on the DUT. | -- |
| 4. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 5. | Verify that the STM is in Normal Operation via the test tool. | Pass or Fail goes here. |
| 6. | Verify that the DUT’s display goes to the main Dashboard screen (with setpoint). | Pass or Fail goes here. |
| 7. | Power cycle the DUT. | -- |
| 8. | IMPORTANT: Immediately connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 9. | Verify that after ~12 seconds, the DUT leaves the loading bar screen and shows the Diagnostic Communication Failure screen. | Pass or Fail goes here. |
| 9. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 10. | Verify that the DUT’s display leaves the Failure screen and goes back to the loading bar screen. | Pass or Fail goes here. |
| 11. | Verify that the DUT’s display goes to the main Dashboard screen. | Pass or Fail goes here. |

### Normal Operation (runtime) - M2M communication loss.

This test will verify that during normal operation, when the CEL loses communication with the STM, the DUT appropriately displays that any data sourced from the STM is unavailable. If communication is re-established, the DUT will go back to normal operation and correctly display data sourced from the STM.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication.

- STM programmed with software that is in Normal Operation.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC.

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the STM32 programmer to your PC and to the correct header on the controller. | -- |
| 2. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 3. | Power on the DUT. | -- |
| 4. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 5. | Verify that the STM is in Normal Operation via the test tool. | Pass or Fail goes here. |
| 6. | Verify that the DUT’s display goes to the main Dashboard screen (with setpoint). | Pass or Fail goes here. |
| 7. | Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 9. | Verify that after ~12-20 seconds, the DUT’s Display now shows “---” (dashes) wherever data from the STM is shown. - Setpoints (Home screen, Temperature submenu screens). - Heating Modes (Home screen, Heating Mode screens). - Temperature Sensors (System Status screen). - Relay states (System Status screen). | Pass or Fail goes here. |
| 9. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 10. | Verify that the DUT’s display goes back to displaying data from the STM. - Setpoints (Home screen, Temperature submenu screens). - Heating Modes (Home screen, Heating Mode screens). - Temperature Sensors (System Status screen). - Relay states (System Status screen). | Pass or Fail goes here. |
| 11. | On the DUT’s display, switch users to pro. - Menu > Switch Users - Enter correct PIN. | -- |
| 12. | Navigate to the Menu > Settings > Pro Tools and enter the Fan Speed selection screen. | -- |
| 13. | Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 14. | Verify that the DUT’s Display exits the Pro Tools settings. | Pass or Fail goes here. |
| 15. | Verify that the DUT is now unable to enter Pro Tools. | Pass or Fail goes here |
| 16. | Disconnect the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 17. | Verify that Pro Tools settings can be accessed again. | Pass or Fail goes here. |
| 18. | Re-do the Lost Communication test in Pro Tools while attempting to access different screens within Pro Tools settings. - EEV - System Status - Etc. | -- Any notes may go here. |

### OTA update (“lockup” STM) - M2M communication loss.

This test will verify that a special diagnostic screen is displayed on powerup when the CEL loses communication with the STM during the OTA process. If communication is re-established while on this diagnostic screen, the display will transition back to the correct workflow.

Estimated Time to Complete: 20 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication screen.

- STM programmed with software that is in Normal Operation.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC.

- MQTT explorer application is installed on your PC and you can view the DUT in this application.

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 2. | Power on the DUT. | -- |
| 3. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 4. | Verify that the STM is in Normal Operation via the test tool. | Pass or Fail goes here |
| 5. | Verify that the DUT’s display goes to the main Dashboard screen (with setpoint). | Pass or Fail goes here |
| 6. | On the Test Tool or Display, verify that the DUT is NOT attempting to heat water. - All relays should be off. | Pass or Fail goes here |
| 7. | Generate an OTA release for the DUT. | -- |
| 8. | On the MQTT explorer, find the DUT and send it a command to look for an OTA. - Note, this is only for Dev units. | -- |
| 9. | Verify that the OTA process begins. | Pass or Fail goes here |
| 10. | Wait for the Display to transition to the OTA loading bar screen. | -- |
| 11. | After the loading bar gets to ~25%, Connect to the STM micro via the STM32CubeProgrammer application on your PC. | -- |
| 12. | Wait for the CEL to reset itself | -- |
| 13. | Verify that after ~12 seconds, the Display transitions to the Lost Communication screen. | Pass or Fail goes here |
| 14. | Disconnect from the STM micro via the STM32CubeProgrammer application on your PC. |  |
| 15. | Power cycle the DUT. |  |
| 16. | Verify that after ~12 seconds, the Display transitions to the Lost Communication screen. |  |
| 14. | Wait for the CEL to recover the STM. - Should take ~2 - 5 minutes. - If you can view the DUT on MQTT explorer, you may monitor the DUT stepping thru the OTA process. | -- |
| 15. | Verify that the DUT’s display transitions back to the “Normal Operation” screens. - Note that you may see an OTA failed screen that can be dismissed. This is expected since the previous OTA failed prior to it being recovered. | Pass or Fail goes here |
| 16. | Verify that the STM is normal operation via the Test Tool. | Pass or Fail goes here |
| 17. | On the DUT’s display, navigate to the Menu > Settings > About Device. | -- |
| 18. | Verify that the software versions are as expected. | Pass or Fail goes here |

### OTA update (power loss) - M2M communication loss.

This test will verify that a special diagnostic screen is displayed on powerup when the CEL loses communication with the STM during the OTA process. If communication is re-established while on this diagnostic screen, the display will transition back to the correct workflow.

Estimated Time to Complete: 20 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- The DUT is powered off.

- CEL programmed with software supports the Lost Communication screen.

- STM programmed with software that is in Normal Operation.

- Two Hearted Test tool is installed on your PC.

- MQTT explorer application is installed on your PC and you can view the DUT in this application.

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 2. | Power on the DUT. | -- |
| 3. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 4. | Verify that the STM is in Normal Operation via the test tool. | Pass or Fail goes here. |
| 5. | Verify that the DUT’s display goes to the main Dashboard screen (with setpoint). | Pass or Fail goes here. |
| 6. | On the Test Tool or Display, verify that the DUT is NOT attempting to heat water. - All relays should be off. | Pass or Fail goes here. |
| 7. | Generate an OTA release for the DUT. | -- |
| 8. | On the MQTT explorer, find the DUT and send it a command to look for an OTA. - Note, this is only for Dev units. | -- |
| 9. | Verify that the OTA process begins. | Pass or Fail goes here. |
| 10. | Wait for the Display to transition to the OTA loading bar screen. | -- |
| 11. | After the loading bar gets to ~25%, power off the DUT. | -- |
| 12. | Wait ~5 seconds and then power on the DUT. | -- |
| 13. | Verify that after ~12 seconds, the Display transitions to the Lost Communication screen. | Pass or Fail goes here. |
| 14. | Wait for the CEL to recover the STM. - Should take ~2 - 5 minutes. - If you can view the DUT on MQTT explorer, you may monitor the DUT stepping thru the OTA process. | -- |
| 15. | Verify that the DUT’s display transitions back to the “Normal Operation” screens. - Note that you may see an OTA failed screen that can be dismissed. This is expected since the previous OTA failed prior to it being recovered. | Pass or Fail goes here. |
| 16. | Verify that the STM is normal operation via the Test Tool. | Pass or Fail goes here. |
| 17. | On the DUT’s display, navigate to the Menu > Settings > About Device. | -- |
| 18. | Verify that the software versions are as expected. | Pass or Fail goes here. |

## Fault notifications

The following test(s) will verify that Faults are correctly displayed onscreen. The tests include basic interaction functions such as pressing the keys on the front face, navigating menus, and changing configurable settings. The tests also include more involved interactions, such as using the ImageTool (CEL) and STM32CubeProgrammer (STM) to flash and erase code. The TwoHearted Test Tool will also be utilized. And lastly, but most critically, unplugging certain connectors and cables from the back of the control module.

### Single Active Fault

This test will verify that a single fault is correctly displayed onscreen when active and no longer displayed when not active.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- There are no active faults.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 2. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 3. | Verify that there are no Displayable Active Errors on the STM via the test tool. | Pass or Fail goes here. |
| 4. | Navigate to the Dashboard Home screen on the DUT’s display. | -- |
| 5. | Verify that the Dashboard Home screen (and Mode and Connectivity screen) only display 3 ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 6. | Verify that there are only 3 Dashboard screens available (consisting of Home, Mode, and Connectivity). | Pass or Fail goes here. |
| 7. | Generate a single Active Fault that can be cleared during runtime. - This may be done in a multitude of ways. But generating a communications fault by connecting the STm32CubeProgrammer to the ST micro and waiting up to 10-20 seconds is probably the easiest. | -- |
| 8. | Verify that the Dashboard Home screen displays that there is a Single Active Fault. | Pass or Fail goes here. |
| 9. | Verify that all Dashboard screens now display 4 ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 10. | Verify that the Dashboard Fault Notification screen is now available, and it displays 4 ellipses. | Pass or Fail goes here. |
| 11. | Verify that the Dashboard Fault Notification screen correctly displays that there is a single active fault and the fault code associated with the single active fault. | Pass or Fail goes here. |
| 12. | Verify that the Active Fault list screen shows only the previously generated active fault. | Pass or Fail goes here. |
| 13. | Verify that the Active Fault list screen shows the scrollbar as one solid color. | Pass or Fail goes here. |
| 14. | Select the active fault to view the fault details screen. | -- |
| 15. | Verify that the Fault Details screen displays the correct fault information and QR code. | Pass or Fail goes here. |
| 16. | While on the fault details screen, clear the active fault. - If you generated a communications fault like previously described, simply disconnect the STM32CubeProgrammer application from the ST micro. | -- |
| 17. | Verify that the Fault Details screen still displays the previously generated active fault information. | Pass or Fail goes here. |
| 18. | Go back to the Active Fault list screen. | -- |
| 19. | Verify that the Active Fault list screen displays no active faults. | Pass or Fail goes here. |
| 20. | Verify that the scrollbar is no longer shown. | Pass or Fail goes here. |
| 21. | While on the Active Fault list screen, generate another fault that can be cleared during runtime - Such as another communication fault. | -- |
| 22. | Verify that the Active Fault list screen properly updates to display the active fault and the scrollbar again. | Pass or Fail goes here. |
| 23. | While on the fault list screen, clear the active fault. | -- |
| 24. | Verify that the fault list screen updates to display no faults and remove the scrollbar. | Pass or Fail goes here. |
| 25. | Go back to the Dashboard Fault Notification screen. | -- |
| 26. | Verify that the Dashboard Fault Notification screen correctly displays that there are no active faults. | Pass or Fail goes here. |
| 27. | While on the Dashboard Fault Notification screen, generate another fault that can be cleared during runtime. | -- |
| 28. | Verify that the Dashboard Fault Notification screen correctly updates to display the single active fault. | Pass or Fail goes here. |
| 29. | While on the Dashboard Fault Notification screen, clear the active fault. | -- |
| 30. | Verify that the Dashboard Fault Notification screen correctly updates to display no active faults. | Pass or Fail goes here. |
| 31. | Navigate to the Dashboard home screen. | -- |
| 32. | Verify that the Dashboard Home screen (and Mode and Connectivity screen) only display 3 ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 33. | Verify that there are only 3 Dashboard screens available again (consisting of Home, Mode, and Connectivity). | Pass or Fail goes here. |
| 34. | While on the Dashboard home screen, generate another active fault that can be cleared. | -- |
| 35. | Verify that the Dashboard home screen correctly displays the active fault, and there are 4 ellipses. | Pass or Fail goes here. |
| 36. | While on the Dashboard home screen, clear the active fault. | -- |
| 37. | Verify that the Dashboard home screen correctly updates to no longer display the active fault, and there are only 3 ellipses. | Pass or Fail goes here. |

### Multiple Active Faults

This test will verify that Multiple Faults are correctly displayed onscreen when active and no longer displayed when not active.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- There are no active faults.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 2. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 3. | Verify that there are no Displayable Active Errors on the STM via the test tool. | Pass or Fail goes here. |
| 4. | Navigate to the Dashboard Home screen on the DUT’s display. | -- |
| 5. | Verify that the Dashboard Home screen (and Mode and Connectivity screen) only display 3 ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 6. | Verify that there are only 3 Dashboard screens available (consisting of Home, Mode, and Connectivity). | Pass or Fail goes here. |
| 7. | Generate multiple Active Faults that can be cleared during runtime. - This may be done in a multitude of ways. Such as unplugging the wiring harness from the back of the controller. | -- |
| 8. | Verify that the Dashboard Home screen displays that there are multiple active faults. | Pass or Fail goes here. |
| 9. | Verify that all Dashboard screens now display 4 ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 10. | Verify that the Dashboard Fault Notification screen is now available, and it displays 4 ellipses. | Pass or Fail goes here. |
| 11. | Verify that the Dashboard Fault Notification screen correctly displays that there are multiple active faults and that the fault codes are correct. | Pass or Fail goes here. |
| 12. | Verify that the Active Fault list screen shows the active faults that are expected. | Pass or Fail goes here. |
| 13. | Verify that the Active Fault list screen shows the scrollbar with a highlight that is proportional to the number of faults in the list. | Pass or Fail goes here. |
| 14. | Verify that the list can be navigated via the Up and Down arrow keys, and the scrollbar highlight correctly updates to reflect this navigation. | Pass or Fail goes here. |
| 15. | Verify that selecting each of the active faults leads to the fault details screen with the correct fault information and QR code. | Pass or Fail goes here. |
| 16. | While on the fault details screen, clear the active faults. | -- |
| 17. | Verify that the Fault Details screen still displays the previously generated active fault information. | Pass or Fail goes here. |
| 18. | Go back to the Active Fault list screen. | -- |
| 19. | Verify that the Active Fault list screen displays no active faults. | Pass or Fail goes here. |
| 20. | Verify that the scrollbar is no longer shown. | Pass or Fail goes here. |
| 21. | While on the Active Fault list screen, generate multiple active faults again that can be cleared during runtime. | -- |
| 22. | Verify that the Active Fault list screen properly updates to display the active faults and the scrollbar again. | Pass or Fail goes here. |
| 23. | While on the fault list screen, clear the active faults. | -- |
| 24. | Verify that the fault list screen updates to display no faults and no scrollbar. | Pass or Fail goes here. |
| 25. | Go back to the Dashboard Fault Notification screen. | -- |
| 26. | Verify that the Dashboard Fault Notification screen correctly displays that there are no active faults. | Pass or Fail goes here. |
| 27. | While on the Dashboard Fault Notification screen, generate multiple active faults again that can be cleared during runtime. | -- |
| 28. | Verify that the Dashboard Fault Notification screen correctly updates to display the multiple active faults. | Pass or Fail goes here. |
| 29. | While on the Dashboard Fault Notification screen, clear the active faults. | -- |
| 30. | Verify that the Dashboard Fault Notification screen correctly updates to display no active faults. | Pass or Fail goes here. |
| 31. | Navigate to the Dashboard home screen. | -- |
| 32. | Verify that the Dashboard Home screen (and Mode and Connectivity screen) only display 3 ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 33. | Verify that there are only 3 Dashboard screens available again (consisting of Home, Mode, and Connectivity). | Pass or Fail goes here. |
| 34. | While on the Dashboard home screen, generate multiple active faults again that can be cleared. | -- |
| 35. | Verify that the Dashboard home screen correctly displays the active fault, and there are 4 ellipses. | Pass or Fail goes here. |
| 36. | While on the Dashboard home screen, clear the active faults. | -- |
| 37. | Verify that the Dashboard home screen correctly updates to no longer displays multiple active faults, and there are only 3 ellipses. | Pass or Fail goes here. |

### Single Historical Fault

This test will verify that a single historical fault is correctly displayed onscreen.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- There are no historical or active faults.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 2. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 3. | Verify that there are no Displayable Active Errors on the STM via the test tool. | Pass or Fail goes here. |
| 4. | Navigate to the Dashboard Home screen on the DUT’s display. | -- |
| 5. | Verify that the Dashboard Home screen (and Mode and Connectivity screen) only display 3 ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 6. | Verify that there are only 3 Dashboard screens available (consisting of Home, Mode, and Connectivity). | Pass or Fail goes here. |
| 7. | Navigate to the Fault History (Menu > History > Fault History). | -- |
| 8. | Verify that the Fault History list shows no faults and no scrollbar. | Pass or Fail goes here. |
| 9. | Generate a single Active Fault that can be cleared during runtime. - This may be done in a multitude of ways. But generating a communications fault by connecting the STm32CubeProgrammer to the ST micro and waiting up to 10-20 seconds is probably the easiest. | -- |
| 10. | Verify that the Fault History list screen updates to show the generated fault with the correct date and time. | Pass or Fail goes here. |
| 11. | Verify that the Fault History list screen shows the scrollbar as one solid color. | Pass or Fail goes here. |
| 12. | Verify that the Fault History list screen shows the correct date and time associated with the generated fault. | Pass or Fail goes here. |
| 14. | Select the fault to view the fault details screen. | -- |
| 15. | Verify that the Fault History Details screen displays the correct fault information and QR code. - This information includes date and time of last occurrence, number of occurrences (1), fault code, and fault description. | Pass or Fail goes here. |
| 16. | While on the fault details screen, clear the active fault. - If you generated a communications fault like previously described, simply disconnect the STM32CubeProgrammer application from the ST micro. | -- |
| 17. | Verify that the fault details screen still displays the previously generated active fault information. | Pass or Fail goes here. |
| 18. | Go back to the Fault History list screen. | -- |
| 19. | Verify that the Fault History list screen still displays the previously generated fault. | Pass or Fail goes here. |
| 20. | Verify that the scrollbar is still shown. | Pass or Fail goes here. |
| 21. | Verify that the Fault History list screen shows the updated date and time associated with the time at which the generated fault was cleared. | Pass or Fail goes here. |
| 22. | Select the fault to view fault details. | -- |
| 23. | Verify the fault details screen shows the same updated date and time associated with when the generated fault was cleared. | Pass or Fail goes here. |
| 24. | Go back to the Fault History list screen. | -- |
| 25. | While on the Fault History list screen, generate the same Fault again. | -- |
| 26. | Verify that the Fault History list screen properly updates to display the date and time associated with when the fault was generated again. | Pass or Fail goes here. |
| 27. | While on the fault list screen, clear the fault again. | -- |
| 28. | Verify that the Fault History list screen properly updates to display the date and time associated with when the fault was cleared again. | Pass or Fail goes here. |
| 29. | Go back to the Dashboard screens. | -- |
| 30. | Verify that the Dashboard screens correctly display no active faults. | Pass or Fail goes here. |

### Multiple Historical Faults

This test will verify that Multiple Historical Faults are correctly displayed onscreen.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- There are no active faults.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 2. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 3. | Verify that there are no Displayable Active Errors on the STM via the test tool. | Pass or Fail goes here. |
| 4. | Navigate to the Dashboard Home screen on the DUT’s display. | -- |
| 5. | Verify that the Dashboard Home screen (and Mode and Connectivity screen) only display 3 ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 6. | Verify that there are only 3 Dashboard screens available (consisting of Home, Mode, and Connectivity). | Pass or Fail goes here. |
| 7. | Navigate to the Fault History (Menu > History > Fault History). | -- |
| 8. | Verify that the Fault History list shows no faults and no scrollbar. | Pass or Fail goes here. |
| 9. | Generate multiple Active Faults that can be cleared during runtime. - This may be done in a multitude of ways. Such as unplugging the wiring harness from the back of the controller. | -- |
| 10. | Verify that the Fault History list screen updates to show the generated faults with the correct dates and times. | Pass or Fail goes here. |
| 11. | Verify that the Fault History list screen shows the scrollbar with a highlight that is proportional to the number of faults in the list. | Pass or Fail goes here. |
| 12. | Verify that the list can be navigated via the Up and Down arrow keys, and the scrollbar highlight changes position to reflect this navigation. | Pass or Fail goes here. |
| 13. | Select each fault and verify that the Fault History Details screen displays the correct fault information and QR code for each fault. - This information includes date and time of last occurrence, number of occurrences (1), fault code, and fault description. | Pass or Fail goes here. |
| 14. | While on one of the fault details screens, clear the active faults. | -- |
| 15. | Verify that the fault details screen still displays the previously generated fault information. | Pass or Failt goes here. |
| 16. | Go back to the Fault History list screen. | -- |
| 17. | Verify that the Fault History list screen still displays the previously generated faults, and the associated dates and times now reflect the date and time at which they were just cleared. | Pass or Fail goes here. |
| 18. | Verify that the Fault History list screen shows the scrollbar with a highlight that is proportional to the number of faults in the list. | Pass or Fail goes here. |
| 19. | Verify that the list can be navigated via the Up and Down arrow keys, and the scrollbar highlight correctly updates to reflect this navigation. | Pass or Fail goes here. |
| 20. | Select each fault and verify that the Fault History Details screen displays the updated fault information and QR code for each fault. | Pass or Fail goes here. |
| 21. | Go back to the Fault History list screen. | -- |
| 22. | While on the Fault History list screen, generate the same faults again. | -- |
| 23. | Verify that the Fault History list screen properly updates to display the new dates and times associated with when the faults were generated again. | Pass or Fail goes here. |
| 24. | While on the fault list screen, clear the active faults again. | -- |
| 25. | Verify that the Fault History list screen properly updates to display the new dates and times associated with when the faults were cleared again. | Pass or Fail goes here. |
| 26. | Go back to the Dashboard screens. | -- |
| 27. | Verify that the Dashboard screens correctly display no active faults. | Pass or Fail goes here. |

## Alert notifications

The following test(s) will verify that Alerts are correctly displayed onscreen. The tests include basic interaction functions such as pressing the keys on the front face, navigating menus, and changing configurable settings. The tests also include more involved interactions, such as using the ImageTool (CEL) and STM32CubeProgrammer (STM) to flash and erase code. The TwoHearted Test Tool will also be utilized. And lastly, but most critically, unplugging certain connectors and cables from the back of the control module.

### Single Active Alert

This test will verify that a single alert is correctly displayed onscreen when active and no longer displayed when not active.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- There are no active alerts.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 2. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 3. | Verify that there are no Displayable Active Errors on the STM via the test tool. | Pass or Fail goes here. |
| 4. | Navigate to the Dashboard Home screen on the DUT’s display. | -- |
| 5. | Verify that the Dashboard screens display the correct set of ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 6. | Verify that the Dashboard Alert notification screen is not available. | Pass or Fail goes here. |
| 7. | Generate a single Active Alert that can be cleared during runtime. | -- |
| 8. | Verify that the Dashboard Home screen displays that there is a Single Active Alert. | Pass or Fail goes here. |
| 9. | Verify that all Dashboard screens display the correct set of ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 10. | Verify that the Dashboard Alert Notification screen is now available, and it displays the correct set of ellipses. | Pass or Fail goes here. |
| 11. | Verify that the Dashboard Alert Notification screen correctly displays that there is a single active Alert and the Alert code associated with the single active Alert. | Pass or Fail goes here. |
| 12. | Verify that the Active Alert list screen shows only the previously generated active Alert. | Pass or Fail goes here. |
| 13. | Verify that the Active Alert list screen shows the scrollbar as one solid color. | Pass or Fail goes here. |
| 14. | Select the active Alert to view the Alert details screen. | -- |
| 15. | Verify that the Alert Details screen displays the correct Alert information and QR code. | Pass or Fail goes here. |
| 16. | While on the Alert details screen, clear the active Alert. | -- |
| 17. | Verify that the Alert Details screen still displays the previously generated active Alert information. | Pass or Fail goes here. |
| 18. | Go back to the Active Alert list screen. | -- |
| 19. | Verify that the Active Alert list screen displays no active alerts. | Pass or Fail goes here. |
| 20. | Verify that the scrollbar is no longer shown. | Pass or Fail goes here. |
| 21. | While on the Active Alert list screen, generate another Alert that can be cleared during runtime | -- |
| 22. | Verify that the Active Alert list screen properly updates to display the active Alert and the scrollbar again. | Pass or Fail goes here. |
| 23. | While on the Alert list screen, clear the active Alert. | -- |
| 24. | Verify that the Alert list screen updates to display no alerts and remove the scrollbar. | Pass or Fail goes here. |
| 25. | Go back to the Dashboard Alert Notification screen. | -- |
| 26. | Verify that the Dashboard Alert Notification screen correctly displays that there are no active alerts. | Pass or Fail goes here. |
| 27. | While on the Dashboard Alert Notification screen, generate another Alert that can be cleared during runtime. | -- |
| 28. | Verify that the Dashboard Alert Notification screen correctly updates to display the single active Alert. | Pass or Fail goes here. |
| 29. | While on the Dashboard Alert Notification screen, clear the active Alert. | -- |
| 30. | Verify that the Dashboard Alert Notification screen correctly updates to display no active alerts. | Pass or Fail goes here. |
| 31. | Navigate to the Dashboard home screen. | -- |
| 32. | Verify that the Dashboard screens display the correct set of ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 33. | Verify that the Dashboard Alert notification screen is not available. | Pass or Fail goes here. |
| 34. | While on the Dashboard home screen, generate another active Alert that can be cleared. | -- |
| 35. | Verify that the Dashboard home screen correctly displays the active Alert, and the correct set of ellipses. | Pass or Fail goes here. |
| 36. | While on the Dashboard home screen, clear the active Alert. | -- |
| 37. | Verify that the Dashboard home screen correctly updates to no longer display the active Alert. | Pass or Fail goes here. |

### Multiple Active Alerts

This test will verify that Multiple Alerts are correctly displayed onscreen when active and no longer displayed when not active.

Estimated Time to Complete: 10 minutes

| Date |  |
| --- | --- |
| Tester |  |
| Test Station |  |
| Unit Serial # |  |
| Test Tool version |  |
| STM software version |  |
| CEL software version |  |
| Overall Test Result | Pass or Fail goes here |

Preconditions:

- There are no active alerts.

- Two Hearted Test tool is installed on your PC.

- STM32CubeProgrammer application is installed your PC

- Necessary cables/programmers are on hand.

| Step | Action | Result (Pass / Fail |
| --- | --- | --- |
| 1. | Connect the test tool cable (RS485) to your PC and to the correct header on the controller. | -- |
| 2. | Connect the Two Hearted Test Tool application on your PC. | -- |
| 3. | Verify that there are no Displayable Active Errors on the STM via the test tool. | Pass or Fail goes here. |
| 4. | Navigate to the Dashboard Home screen on the DUT’s display. | -- |
| 5. | Verify that the Dashboard screens display the correct set of ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 6. | Verify that the Dashboard Alert notification screen is not available. | Pass or Fail goes here. |
| 7. | Generate multiple Active Alerts that can be cleared during runtime. - This may be done in a multitude of ways. Such as unplugging the wiring harness from the back of the controller. | -- |
| 8. | Verify that the Dashboard Home screen displays that there are multiple active alerts. | Pass or Fail goes here. |
| 9. | Verify that all Dashboard screens now display the correct set of ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 10. | Verify that the Dashboard Alert Notification screen is now available, and it displays the correct set of ellipses. | Pass or Fail goes here. |
| 11. | Verify that the Dashboard Alert Notification screen correctly displays that there are multiple active alerts and that the Alert codes are correct. | Pass or Fail goes here. |
| 12. | Verify that the Active Alert list screen shows the active alerts that are expected. | Pass or Fail goes here. |
| 13. | Verify that the Active Alert list screen shows the scrollbar with a highlight that is proportional to the number of alerts in the list. | Pass or Fail goes here. |
| 14. | Verify that the list can be navigated via the Up and Down arrow keys, and the scrollbar highlight correctly updates to reflect this navigation. | Pass or Fail goes here. |
| 15. | Verify that selecting each of the active alerts leads to the Alert details screen with the correct Alert information and QR code. | Pass or Fail goes here. |
| 16. | While on the Alert details screen, clear the active alerts. | -- |
| 17. | Verify that the Alert Details screen still displays the previously generated active Alert information. | Pass or Fail goes here. |
| 18. | Go back to the Active Alert list screen. | -- |
| 19. | Verify that the Active Alert list screen displays no active alerts. | Pass or Fail goes here. |
| 20. | Verify that the scrollbar is no longer shown. | Pass or Fail goes here. |
| 21. | While on the Active Alert list screen, generate multiple active alerts again that can be cleared during runtime. | -- |
| 22. | Verify that the Active Alert list screen properly updates to display the active alerts and the scrollbar again. | Pass or Fail goes here. |
| 23. | While on the Alert list screen, clear the active alerts. | -- |
| 24. | Verify that the Alert list screen updates to display no alerts and no scrollbar. | Pass or Fail goes here. |
| 25. | Go back to the Dashboard Alert Notification screen. | -- |
| 26. | Verify that the Dashboard Alert Notification screen correctly displays that there are no active alerts. | Pass or Fail goes here. |
| 27. | While on the Dashboard Alert Notification screen, generate multiple active alerts again that can be cleared during runtime. | -- |
| 28. | Verify that the Dashboard Alert Notification screen correctly updates to display the multiple active alerts. | Pass or Fail goes here. |
| 29. | While on the Dashboard Alert Notification screen, clear the active alerts. | -- |
| 30. | Verify that the Dashboard Alert Notification screen correctly updates to display no active alerts. | Pass or Fail goes here. |
| 31. | Navigate to the Dashboard home screen. | -- |
| 32. | Verify that the Dashboard screens display the correct set of ellipses, located just above the Toolbar at the bottom of the screen. | Pass or Fail goes here. |
| 33. | Verify that the Dashboard Alert notification screen is not available. | Pass or Fail goes here. |
| 34. | While on the Dashboard home screen, generate multiple active alerts again that can be cleared. | -- |
| 35. | Verify that the Dashboard home screen correctly displays the active Alert, and the correct set of ellipses. | Pass or Fail goes here. |
| 36. | While on the Dashboard home screen, clear the active alerts. | -- |
| 37. | Verify that the Dashboard home screen correctly updates to no longer displays multiple active alerts. | Pass or Fail goes here. |

## Final Assembly Testing (FAT) Mode

- Run through FAT mode on the factory floor at the repair station

- Test 50 gallon, 65 gallon, 80 gallon

## User Settings (CEL) Testing

- Display

  - Language Setting

  - Brightness

  - Screen Timeout

  - Lock Screen

  - Status Screen

    - temp display

    - element state

    - RMS voltage

  - Wifi Icon status

- Date / Time / Location

  - Change date/time/timezone/dst

## Constants Table

### Hybrid

| 50 gal | SA | -1 | CSA | 51 | CUA | 3 | CTA | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 46 | CSB | 54 | CUB |  | CTB | 22 |
|  | SC | 5 | CSC | 5 |  |  | CTC | 5 |
|  | SD | 7 | CSD |  |  |  | CTD |  |
|  |  |  | CSE | 16 |  |  | CTE | 12 |
|  |  |  | CSF | 300 |  |  | CTF | 300 |

| 65 gal | SA | -1 | CSA | 42 | CUA | 2.5 | CTA | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 37 | CSB | 45 | CUB |  | CTB | 22 |
|  | SC | 4 | CSC | 5 |  |  | CTC | 5 |
|  | SD | 6 | CSD |  |  |  | CTD |  |
|  |  |  | CSE | 25 |  |  | CTE | 23 |
|  |  |  | CSF | 300 |  |  | CDF | 300 |

| gal | SA | -1 | CSA | 32 | CUA | 1.5 | CTA | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 27 | CSB | 30 | CUB |  | CTB | 22 |
|  | SC | 4 | CSC | 5 |  |  | CTC | 5 |
|  | SD | 6 | CSD |  |  |  | CTD |  |
|  |  |  | CSE | 14 |  |  | CTE | 14 |
|  |  |  | CSF | 300 |  |  | CDF | 300 |

### Hybrid Plus

| 50gal | SA | -1 | CSA | 7 | CLA | 20 | CUA | 7 | CTA | 0.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 20 | CSB | 20 | CLB |  | CUB |  | CTB | 18 |
|  | SC | 4 | CSC | 5 | CLC | 4 |  |  | CTC | 5 |
|  | SD | 6 | CSD |  | CLD | 7 |  |  | CTD | 10 |

| 65 gal | SA | -1 | CSA | 6 | CLA | 15 | CUA | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 15 | CSB | 15 | CLB |  | CUB |  |
|  | SC | 4 | CSC | 5 | CLC | 2 |  |  |
|  | SD | 6 | CSD |  | CLD | 7 |  |  |

| 80 gal | SA | -1 | CSA | 3 | CLA | 13 | CUA | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 13 | CSB | 13 | CLB |  | CUB |  |
|  | SC | 4 | CSC | 5 | CLC | 2 |  |  |
|  | SD | 6 | CSD |  | CLD | 7 |  |  |

### Heat Pump Only

| 50gal | SA | -1 | CSA | 7 |
| --- | --- | --- | --- | --- |
|  | SB | 20 | CSB | 20 |
|  | SC | 4 | CSC | 5 |
|  | SD | 6 | CSD |  |

| 65 gal | SA | -1 | CSA | 6 |
| --- | --- | --- | --- | --- |
|  | SB | 15 | CSB | 15 |
|  | SC | 4 | CSC | 5 |
|  | SD | 6 | CSD |  |

| 80 gal | SA | -1 | CSA | 3 |
| --- | --- | --- | --- | --- |
|  | SB | 13 | CSB | 13 |
|  | SC | 4 | CSC | 5 |
|  | SD | 6 | CSD |  |

### Heat Pump No Electric

| 50gal | SA | -1 |
| --- | --- | --- |
|  | SB | 20 |
|  | SC | 4 |
|  | SD | 6 |

| 65 gal | SA | -1 |
| --- | --- | --- |
|  | SB | 15 |
|  | SC | 4 |
|  | SD | 6 |

| 80 gal | SA | -1 |
| --- | --- | --- |
|  | SB | 13 |
|  | SC | 4 |
|  | S | 6 |

### Electric

| 50gal | SA | 0.1 | UA | 7 | LA | 10 | FA | 0.5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 17 | UB |  | LB | 22 | FB | 5 |  |  |
|  | SC |  | UC |  | LC |  | FC |  |  |  |
|  | SC |  | UC |  | LC |  | FD | 22 |  |  |

| 65 gal | SA | 0.1 | UA | 7 | LA | 10 | FA | 0.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 17 | UB |  | LB | 22 | FB | 5 |
|  | SC |  | UC |  | LC |  | FC |  |
|  | SC |  | UC |  | LC |  | FD | 22 |

| 80 gal | SA | 0.1 | UA | 7 | LA | 10 | FA | 0.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 17 | UB |  | LB | 22 | FB | 5 |
|  | SC |  | UC |  | LC |  | FC |  |
|  | SC |  | UC |  | LC |  | FD | 22 |

### Vacation

| 50gal | SA |  |
| --- | --- | --- |
|  | SB |  |
|  | SC | 4 |
|  | SD | 4 |

| 65 gal | SA |  |
| --- | --- | --- |
|  | SB |  |
|  | SC | 4 |
|  | SD | 4 |

| 80 gal | SA |  |
| --- | --- | --- |
|  | SB |  |
|  | SC | 4 |
|  | S | 4 |

### Hybrid Lower

| 50gal | SA | -1 | CSA | 3 | CLA | 6 | CTA | 0.1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 27 | CSB | 27 | CLB |  | CTB | 22 |  |  |
|  | SC | 4 | CSC | 5 | CLC | 1 | CTC | 5 |  |  |
|  | SD | 6 | CSD |  | CLD |  | CTD | 10 |  |  |

| 65 gal | SA | -1 | CSA | 3 | CLA | 6 | CTA | 0.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 27 | CSB | 27 | CLB |  | CTB | 22 |
|  | SC | 4 | CSC | 5 | CLC | 1 | CTC | 5 |
|  | SD | 6 | CSD |  | CLD |  | CTD | 10 |

| 80 gal | SA | -1 | CSA | 3 | CLA | 6 | CTA | 0.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SB | 27 | CSB | 27 | CLB |  | CTB | 22 |
|  | SC | 4 | CSC | 5 | CLC | 1 | CTC | 5 |
|  | SD | 6 | CSD |  | CLD |  | CTD | 10 |

## TODO

A water heater heats water

- It must heat water when below setpoint

- It must not heat water when at setpoint

- It must not heat water when above setpoint

Temperature Ranges

- Minimum temp of 100

- Maximum temp of 140

- Vacation drops min to 54

When do mode changes occur

- Mode change when AT setpoint

- Mode change when BELOW setpoint (while heating)

Setpoint changes during

- Change setpoint while heating

- Change setpoint when AT setpoint

Variations of modes:

- Normal operating modes

  - Hybrid

  - Hybrid Plus

  - Electric

  - Heat Pump

  - Vacation

- CTA Modes

  - Shed

  - Loadup

  - Critical Peak

  - Grid Emergency

- Schedules

  - TOU Schedules

    - Shed

    - Loadup

  - Normal Schedules

    - Hybrid

    - Hybrid Plus

    - Electric

    - Heat Pump

    - Vacation

- Service Mode